## 5.2 MIM EXPRESS short listing

This clause specifies the EXPRESS schema derived from the mapping table. It uses elements from the common resources or from other application modules and defines the EXPRESS constructs that are specific to this part of ISO 10303.

This clause constitutes the Module Interpreted Module (MIM) of the application module.

This clause also specifies the modifications that apply to the constructs imported from the common resources.

The following restrictions apply to the use, in this schema, of constructs defined in common resources or in application modules:

- use of a supertype entity does not make applicable any of its specializations, unless the specialization is also imported in the MIM schema;

- use of a SELECT type does not make applicable any of its listed types unless the listed type is also imported in the MIM schema.

Short names of entities defined in this schema are described in Annex A. Unambiguous identification of this schema is defined in Annex B.

EXPRESS specification: 

*)
SCHEMA Activity_as_realized_mim;

USE FROM action_schema    --  ISO 10303-41
  (action_relationship, 
   executed_action);

USE FROM Activity_mim;    --  ISO/TS 10303-1047
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| action_schema | ISO 10303-41 |
| --- | --- |
| Activity_mim | ISO/TS 10303-1047 |

NOTE 2   See Annex D, Figures D.1and D.2 for a graphical representation of this schema.

## 5.2.1 MIM entity definitions

This subclause specifies the MIM entities for this module. The MIM entities and definitions are specified below.

## 5.2.1.1 action_actual

An action_actual is a type of executed_action. It records the fact that an action is part of the historical record, and has at least started.

EXPRESS specification:

*)
ENTITY action_actual
  SUBTYPE OF (executed_action);
END_ENTITY;
(*

## 5.2.1.2 action_happening

An action_happening is a type of action_relationship. It relates the action_actual to the action of which it is the historical record.

EXPRESS specification:

*)
ENTITY action_happening
  SUBTYPE OF (action_relationship);
END_ENTITY;
(*

*)
END_SCHEMA;  -- Activity_as_realized_mim
(*

© ISO 2019 — All rights reserved