## 4 Information requirements

This clause specifies the information requirements for the Alternative solution application module. The information requirements are specified as the Application Reference Model (ARM) of this application module.

NOTE 1  A graphical representation of the information requirements is given in Annex C.

NOTE 2  The mapping specification is specified in 5.1. It shows how the information requirements are met by using common resources and constructs defined or imported in the MIM schema of this application module.

This clause defines the information requirements to which implementations shall conform using the EXPRESS language as defined in ISO 10303-11. The following begins the Alternative_solution_arm schema and identifies the necessary external references.

EXPRESS specification: 

*)
SCHEMA Alternative_solution_arm;
(*

## 4.1 Required AM ARMs

The following EXPRESS interface statements specify the elements imported from the ARMs of other application modules.

EXPRESS specification:

*)
USE FROM Functional_breakdown_arm;    --  ISO/TS 10303-1216

USE FROM Part_view_definition_arm;    --  ISO/TS 10303-1023

USE FROM Person_organization_assignment_arm;    --  ISO/TS 10303-1013

USE FROM Physical_breakdown_arm;    --  ISO/TS 10303-1215

USE FROM Product_as_individual_arm;    --  ISO/TS 10303-1164

USE FROM Product_occurrence_arm;    --  ISO/TS 10303-1063

USE FROM Requirement_view_definition_arm;    --  ISO/TS 10303-1141
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Functional_breakdown_arm | ISO/TS 10303-1216 |
| --- | --- |
| Part_view_definition_arm | ISO/TS 10303-1023 |
| Person_organization_assignment_arm | ISO/TS 10303-1013 |
| Physical_breakdown_arm | ISO/TS 10303-1215 |
| Product_as_individual_arm | ISO/TS 10303-1164 |
| Product_occurrence_arm | ISO/TS 10303-1063 |
| Requirement_view_definition_arm | ISO/TS 10303-1141 |

NOTE 2   See Annex C, Figures C.1and C.2 for a graphical representation of this schema.

## 4.2 ARM type definitions

This subclause specifies the ARM types for this application module. The ARM types and definitions are specified below.

## 4.2.1 complex_product_select

The complex_product_select type allows for the designation of the data types Alternative_solution, Breakdown_element, Functional_element_definition, Part_view_definition, and Physical_element_definition.

EXPRESS specification:

*)
TYPE complex_product_select = SELECT
   (Alternative_solution, 
    Breakdown_element, 
    Functional_element_definition, 
    Part_view_definition, 
    Physical_element_definition);
END_TYPE; 
(*

## 4.2.2 final_definition_select

The final_definition_select type allows for the designation of the data types Product_as_individual_view, Part_view_definition, and Requirement_view_definition.

EXPRESS specification:

*)
TYPE final_definition_select = SELECT
   (Product_as_individual_view, 
    Part_view_definition, 
    Requirement_view_definition);
END_TYPE; 
(*

## 4.3 ARM entity definitions

This subclause specifies the ARM entities for this module. Each ARM application entity is an atomic element that embodies a unique application concept and contains attributes specifying the data elements of the entity. The ARM entities and definitions are specified below.

## 4.3.1 Alternative_solution

An Alternative_solution is the identification of one of the potentially many mutually exclusive implementations of a Functional_element_definition or a Physical_element_definition.

An Alternative_solution may refer directly to the resolved concept or another Alternative_solution. In the latter case, it specifies a refinement of the referenced Alternative_solution.

Each Alternative_solution may be specialised as a Technical_solution or a Supplier_solution.

EXPRESS specification:

*)
ENTITY Alternative_solution;
  id : STRING;
  version_id : OPTIONAL STRING;
  base_element : complex_product_select;
END_ENTITY;
(*

Attribute definitions:

id: the identifier of the Alternative_solution.

version_id: the version of the Alternative_solution. The version_id need not be specified for a particular Alternative_solution.

base_element: the object for which the Alternative_solution provides a design alternative. All Alternative_solution objects for the same base_element are mutually exclusive.

## 4.3.2 Alternative_solution_relationship

An Alternative_solution_relationship is a relationship between two Alternative_solution objects the meaning of which is defined by the attribute relation_type.

EXPRESS specification:

*)
ENTITY Alternative_solution_relationship;
  description : OPTIONAL STRING;
  id : OPTIONAL STRING;
  related : Alternative_solution;
  relating : Alternative_solution;
  relation_type : STRING;
WHERE
  WR1: relating :<>: related;
END_ENTITY;
(*

Attribute definitions:

description: the text or the set of texts that provide further information about the Alternative_solution_relationship. The value of this attribute need not be specified.

id: the identifier or set of identifiers for the Alternative_solution_relationship.

related: the second of the two Alternative_solution objects related by the Alternative_solution_relationship.

relating: the first of the two Alternative_solution objects related by the Alternative_solution_relationship.

relation_type: the meaning of the relationship. Where applicable, the following values shall be used:

- 'decomposition': The related object is one of potentially more components of the relating Alternative_solution. This relation type shall only be used for relating and related objects of the same type;

- 'functionality': The related object is an element of the functional structure of the relating object;

EXAMPLE    The Alternative_solution 'heat dissipation' may be considered as one of the functionalities to be fulfilled by any Technical_solution. for the Breakdown_element; the 'braking system'.

- 'realization': The related Alternative_solution is a means for fulfilling, either partially or fully, the requirements identified by the relating Alternative_solution. This relation type shall only be used for relating and related objects of different types;

- 'specialization': The related object fulfils the requirements of the relating Alternative_solution in a more specific way than defined for the relating Alternative_solution. This relation type shall only be used for relating and related objects of the same type.

Formal propositions:

WR1: the related and the relating must be different.

## 4.3.3 Final_solution

A Final_solution is a type of Alternative_solution and is a set of additional sensual characteristics that can be applied to an Occurrence that represents a neutral part to complete its definition.

EXPRESS specification:

*)
ENTITY Final_solution
  SUBTYPE OF (Alternative_solution);
  specifications : SET[1:?] OF final_definition_select;
  status : STRING;
END_ENTITY;
(*

Attribute definitions:

specifications: the set of means of finalisation that is applied to the neutral part and which may be objects of type Product_as_individual_view,Part_view_definition or Requirement_view_definition.

status: the text string that describes the level of completion between the neutral part and the final part.

## 4.3.4 Solution_element

A Solution_element is the association of a functional or organic requirement with an occurrence of a part that provides a way to resolve that requirement. The requirement is represented with a Functional_element_definition, a Physical_element_definition or with an Alternative_solution in the case where several parts may resolve the requirement.

EXAMPLE 1   In a commercial aircraft, the function of the main passenger exit is usually realised by the front left door.

NOTE    The requirement may be resolved by one part occurrence or by several part occurrences.

EXAMPLE 2   Closure of a door in a house is usually realised with a lock and a key.

EXPRESS specification:

*)
ENTITY Solution_element;
  resolved_requirement : complex_product_select;
  element : Product_occurrence;
  description : OPTIONAL STRING;
END_ENTITY;
(*

Attribute definitions:

resolved_requirement: the Functional_element_definition, the Physical_element_definition or the Alternative_solution.

element: the part occurrence that provides a full or partial way to resolve the requirement.

description: a text that provides further information about the rationale to consider the element as an element of the solution. The value of this attribute need not be specified.

## 4.3.5 Supplier_solution

A Supplier_solution is a type of Alternative_solution provided by a particular supplier.

EXAMPLE    A Physical_element 'handle' has a Technical_solution 'H34215-VR17'. Three suppliers A, B, and C propose to handle products that conform to that solution. A Supplier_solution can be instantiated to represent the solution proposed by each of these suppliers.

EXPRESS specification:

*)
ENTITY Supplier_solution
  SUBTYPE OF (Alternative_solution);
  supplier : Organization;
  probability_rate : OPTIONAL REAL;
DERIVE
  actual_rate : REAL := NVL(probability_rate, 1.0);
WHERE
  WR1: {0.0 <= actual_rate <= 1.0};
END_ENTITY;
(*

Attribute definitions:

supplier: the Organization that acts as the supplier of the solution.

probability_rate: the share that is assigned to the supplier in the context of the base element. The probability_rate need not be specified for a particular Supplier_solution. In a case where the probability_rate is not specified; it shall be considered that the share granted to the Supplier_solution is either not yet known or not provided by intent.

actual_rate: equal to the probability_rate or, by default, to 1.

NOTE    This derived attribute is mainly specified to constraint the supplier share to be positive.

Formal propositions:

WR1: The actual_rate shall be a value between 0 and 1.

## 4.3.6 Technical_solution

A Technical_solution is a type of Alternative_solution where the functional or organic requirements are fulfilled in a particularly technical way.

EXAMPLE    'gilded handle', 'unpolished handle' are examples of Technical_solution for the Physical_element 'handle'.

EXPRESS specification:

*)
ENTITY Technical_solution
  SUBTYPE OF (Alternative_solution);
  description : STRING;
END_ENTITY;
(*

Attribute definitions:

description: additional information about the Technical_solution.

*)
END_SCHEMA;  -- Alternative_solution_arm
(*

© ISO 2019 — All rights reserved