## 4.1 Required AM ARM

The following EXPRESS interface statement specifies the elements imported from the ARM of another application module.

EXPRESS specification:

*)
USE FROM Assembly_module_design_arm;    --  ISO/TS 10303-1636
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Assembly_module_design_arm | ISO/TS 10303-1636 |
| --- | --- |

NOTE 2   See Annex C, Figures C.1and C.2 for a graphical representation of this schema.

## 4.2 ARM entity definitions

This subclause specifies the ARM entities for this module. Each ARM application entity is an atomic element that embodies a unique application concept and contains attributes specifying the data elements of the entity. The ARM entities and definitions are specified below.

## 4.2.1 Assembly_module_component

An Assembly_module_component is a type of Physical_component and a type of Single_instance. The Assembly_module_component represents the inclusion of a specific product version in the associated assembly.

NOTE 1   Assembly_module_component may be useful in a vertically integrated concurrent engineering environment, where full data sharing is allowed, as it permits visibility into the design definition of the component.

NOTE 2   An enterprise could use this capability in the case of co-design of assembly and integrated circuits.

NOTE 3   Hierarchical physical design does not require this capability, but this is the most explicit way to support hierarchical electrical traceability.

EXAMPLE    Consider a daughter board instance as an Assembly_module_component. The design organization maintains explicit traceability between the design of the motherboard and the design of the daughter board.

EXPRESS specification:

*)
ENTITY Assembly_module_component
  SUBTYPE OF (Physical_component, Single_instance);
  SELF\Definition_based_product_occurrence.derived_from : Assembly_module_design_view;
DERIVE
  usage_view : Assembly_module_usage_view := derived_from\Part_design_view.usage_view;
INVERSE
  access_mechanisms : SET[2:?] OF Assembly_module_component_terminal FOR associated_definition;
WHERE
  WR1: NOT EXISTS(SELF\Product_view_definition.name);
  WR2: usage_view = access_mechanisms[1]\Assembly_module_component_terminal.definition\Assembly_module_terminal.associated_definition;
END_ENTITY;
(*

Attribute definitions:

derived_from: specifies the role of the Assembly_module_design_view for the Assembly_module_component.

usage_view: specifies the role of the Assembly_module_usage_view for the Assembly_module_component.

access_mechanisms: specifies an inverse relationship that specifies that the existence of the Assembly_module_component is dependent on the existence of the Assembly_module_component_terminal that specifies the Assembly_module_component as its associated_definition. There shall be two or more Assembly_module_component_terminal for an Assembly_module_component.

Formal propositions:

WR1: The name shall not be populated.

WR2: The Assembly_module_usage_view referenced by usage_view shall also be referenced by the definition of the Assembly_module_component_terminal that references the Assembly_module_component.

## 4.2.2 Assembly_module_component_terminal

An Assembly_module_component_terminal is a type of Physical_component_terminal. The Assembly_module_component_terminal is included with the Assembly_module_component to assure the data integrity of references to the terminals of the Assembly_module_component.

EXPRESS specification:

*)
ENTITY Assembly_module_component_terminal
  SUBTYPE OF (Physical_component_terminal);
  SELF\Component_feature.associated_definition : Assembly_module_component;
DERIVE
  SELF\Component_feature.definition : Component_path_shape_element := assigned_component_path_shape_element;
INVERSE
  assigned_component_path_shape_element : Component_path_shape_element FOR component_shape_element;
WHERE
  WR1: NOT EXISTS(SELF\Shape_element.description);
  WR2: 'PHYSICAL_COMPONENT_FEATURE_ARM.PHYSICAL_COMPONENT_TERMINAL' IN TYPEOF(definition);
END_ENTITY;
(*

Attribute definitions:

associated_definition: specifies the role of the Assembly_module_component for the Assembly_module_component_terminal.

definition: specifies the role of the Assembly_module_terminal for the Assembly_module_component_terminal.

assigned_component_path_shape_element: specifies an inverse relationship that specifies that the existence of the Assembly_module_component_terminal is dependent on the existence of the Component_path_shape_element that specifies the Assembly_module_component_terminal as its component_shape_element.

Formal propositions:

WR1: The description shall not be populated.

WR2: The definition shall be a Physical_component_feature.

*)
END_SCHEMA;  -- Assembly_module_with_subassembly_arm
(*

© ISO 2019 — All rights reserved