## 4.1 Required AM ARMs

The following EXPRESS interface statements specify the elements imported from the ARMs of other application modules.

EXPRESS specification:

*)
USE FROM Assembly_component_arm;    --  ISO/TS 10303-1802

USE FROM Assembly_shape_arm;    --  ISO/TS 10303-1808
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Assembly_component_arm | ISO/TS 10303-1802 |
| --- | --- |
| Assembly_shape_arm | ISO/TS 10303-1808 |

NOTE 2   See Annex C, Figures C.1and C.2 for a graphical representation of this schema.

## 4.2 ARM type definition

This subclause specifies the ARM type for this application module. The ARM type and definition is specified below.

## 4.2.1 cf_groupable_item

The cf_groupable_item type is an extension of the groupable_item type. It adds the data type Component_feature to the list of alternate data types.

EXPRESS specification:

*)
TYPE cf_groupable_item = SELECT BASED_ON groupable_item WITH 
   (Component_feature);
END_TYPE; 
(*

## 4.3 ARM entity definitions

This subclause specifies the ARM entities for this module. Each ARM application entity is an atomic element that embodies a unique application concept and contains attributes specifying the data elements of the entity. The ARM entities and definitions are specified below.

## 4.3.1 Component_feature

A Component_feature is a type of Shape_element The location of the Component_feature in the assembly model is derived from the placement of the associated_definition in the assembly model and the placement of the definition model in the derived_from model of that associated_definition.

EXPRESS specification:

*)
ENTITY Component_feature
  SUBTYPE OF (Shape_element);
  definition : OPTIONAL Part_shape_element;
  SELF\Shape_element.associated_definition : Assembly_component;
DERIVE
  SELF\Shape_element.product_definitional : BOOLEAN := TRUE;
UNIQUE
  UR1: definition, associated_definition;
  UR2: SELF\Shape_element.element_name, associated_definition;
END_ENTITY;
(*

Attribute definitions:

definition: the General_part_feature that serves as a definition for the Component_feature. The value of this attribute need not be specified.

associated_definition: the Assembly_component that has the Component_feature.

NOTE    The subtypes of Component_feature specify specializations of Assembly_component that are consistent with the subtype of definition.

product_definitional: a Component_feature is always definitional.

Formal propositions:

UR1: The combination of definition and associated_definition shall be unique within a population of Component_feature.

UR2: The combination of element_name and associated_definition shall be unique within a population of Component_feature.

## 4.3.2 Component_feature_joint

A Component_feature_joint is a type of Component_feature_relationship and of Shape_element for the representation of a joint between two Component_features.

EXPRESS specification:

*)
ENTITY Component_feature_joint
  ABSTRACT SUPERTYPE 
  SUBTYPE OF (Component_feature_relationship, Shape_element);
  SELF\Component_feature_relationship.relating RENAMED feature_1 : Component_feature;
  SELF\Component_feature_relationship.related RENAMED feature_2 : Component_feature;
  SELF\Shape_element.associated_definition : Assembly_definition;
WHERE
  WR1: feature_1 <> feature_2;
  WR2: associated_definition IN feature_1\Shape_element.associated_definition\Assembly_component.assemblies;
  WR3: associated_definition IN feature_2\Shape_element.associated_definition\Assembly_component.assemblies;
END_ENTITY;
(*

Attribute definitions:

feature_1: specifies the role of the Component_feature for the Component_feature_joint.

feature_2: specifies the role of the Component_feature for the Component_feature_joint.

associated_definition: specifies the Assembly_definition to which the Component_feature_joint belongs.

Formal propositions:

WR1: the feature_1 and feature_2 shall not point to the same instance of Component_feature.

WR2: the Component_feature referenced by feature_1 shall be used by the same Assembly_definition as the associated_definition.

WR3: the Component_feature referenced by feature_2 shall be used by the same Assembly_definition as the associated_definition.

## 4.3.3 Component_feature_make_from_relationship

A Component_feature_make_from_relationship is a type of Component_feature_relationship. A Component_feature_make_from_relationship is an association between two Component_feature, where one Component_feature is used as a basis for the development of another Component_feature.

EXPRESS specification:

*)
ENTITY Component_feature_make_from_relationship
  SUBTYPE OF (Component_feature_relationship);
  SELF\Shape_element_relationship.relating RENAMED reusable_feature : Component_feature;
  SELF\Shape_element_relationship.related RENAMED resultant_feature : Component_feature;
END_ENTITY;
(*

Attribute definitions:

reusable_feature: specifies the Component_feature that is modified or reused to become the resultant_feature in the Component_feature_make_from_relationship.

resultant_feature: specifies the Component_feature that results from modifying or reusing the reusable_feature in the Component_feature_make_from_relationship.

## 4.3.4 Component_feature_relationship

A Component_feature_relationship is a type of Shape_element_relationship. A Component_feature_relationship is the association between two Component_feature. A Component_feature_relationship may be a Component_feature_joint or a Component_feature_make_from_relationship.

EXPRESS specification:

*)
ENTITY Component_feature_relationship
  SUPERTYPE OF (ONEOF (Component_feature_joint,
                       Component_feature_make_from_relationship))
  SUBTYPE OF (Shape_element_relationship);
  SELF\Shape_element_relationship.relating : Component_feature;
  SELF\Shape_element_relationship.related : Component_feature;
END_ENTITY;
(*

Attribute definitions:

relating: the first instance of Component_feature that is part of the relationship.

related: the second of the instances of Component_feature that is part of the relationship. If one element of the relationship is dependent upon the other, this attribute shall be the dependent one.

## 4.3.5 Component_terminal

A Component_terminal is a type of Component_feature. A Component_terminal is the defined conductive shape for conductors from other components to access the electrical functionality of the Assembly_component.

EXPRESS specification:

*)
ENTITY Component_terminal
  ABSTRACT SUPERTYPE 
  SUBTYPE OF (Component_feature);
  local_swappable : OPTIONAL BOOLEAN;
  global_swappable : OPTIONAL BOOLEAN;
  swap_code : OPTIONAL STRING;
END_ENTITY;
(*

Attribute definitions:

local_swappable: specifies a Boolean value that indicates if the terminal can be exchanged within the associated functional unit. The value of this attribute need not be specified.

global_swappable: specifies a Boolean value that indicates if the terminal can be exchanged within associated functional units of the same type. The value of this attribute need not be specified.

swap_code: specifies a string for the Component_terminal. The value of this attribute need not be specified.

*)
END_SCHEMA;  -- Component_feature_arm
(*

© ISO 2019 — All rights reserved