## 4.1 Required AM ARMs

The following EXPRESS interface statements specify the elements imported from the ARMs of other application modules.

EXPRESS specification:

*)
USE FROM Advanced_boundary_representation_arm;    --  ISO/TS 10303-1514

USE FROM Construction_geometry_arm;    --  ISO/TS 10303-1131

USE FROM Constructive_solid_geometry_3d_arm;    --  ISO/TS 10303-1068

USE FROM Curve_swept_solid_arm;    --  ISO/TS 10303-1659

USE FROM Edge_based_wireframe_arm;    --  ISO/TS 10303-1501

USE FROM Elementary_boundary_representation_arm;    --  ISO/TS 10303-1329

USE FROM External_model_arm;    --  ISO/TS 10303-1033

USE FROM Faceted_boundary_representation_arm;    --  ISO/TS 10303-1512

USE FROM Geometrically_bounded_surface_arm;    --  ISO/TS 10303-1507

USE FROM Geometrically_bounded_wireframe_arm;    --  ISO/TS 10303-1510

USE FROM Manifold_surface_arm;    --  ISO/TS 10303-1509

USE FROM Shell_based_wireframe_arm;    --  ISO/TS 10303-1502

USE FROM Tessellated_geometry_arm;    --  ISO/TS 10303-1819

USE FROM Value_with_unit_extension_arm;    --  ISO/TS 10303-1753
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Advanced_boundary_representation_arm | ISO/TS 10303-1514 |
| --- | --- |
| Construction_geometry_arm | ISO/TS 10303-1131 |
| Constructive_solid_geometry_3d_arm | ISO/TS 10303-1068 |
| Curve_swept_solid_arm | ISO/TS 10303-1659 |
| Edge_based_wireframe_arm | ISO/TS 10303-1501 |
| Elementary_boundary_representation_arm | ISO/TS 10303-1329 |
| External_model_arm | ISO/TS 10303-1033 |
| Faceted_boundary_representation_arm | ISO/TS 10303-1512 |
| Geometrically_bounded_surface_arm | ISO/TS 10303-1507 |
| Geometrically_bounded_wireframe_arm | ISO/TS 10303-1510 |
| Manifold_surface_arm | ISO/TS 10303-1509 |
| Shell_based_wireframe_arm | ISO/TS 10303-1502 |
| Tessellated_geometry_arm | ISO/TS 10303-1819 |
| Value_with_unit_extension_arm | ISO/TS 10303-1753 |

NOTE 2   See Annex C, Figures C.1, C.2, C.3and C.4 for a graphical representation of this schema.

## 4.2 ARM type definitions

This subclause specifies the ARM types for this application module. The ARM types and definitions are specified below.

## 4.2.1 boundary_class_zone

The boundary_class_zone type allows for the designation of the data types Boundary_curve_set, and Loop.

EXPRESS specification:

*)
TYPE boundary_class_zone = SELECT
   (Boundary_curve_set, 
    Loop);
END_TYPE; 
(*

## 4.2.2 composite_assembly_shape

The composite_assembly_shape type allows for the designation of the data types Advanced_brep_shape_representation, Edge_based_wireframe_shape_representation, External_geometric_model, Faceted_brep_shape_representation, Geometrically_bounded_surface_shape_representation, Geometrically_bounded_wireframe_shape_representation, Manifold_surface_shape_representation, Shell_based_wireframe_shape_representation, Tessellated_shape_representation, and Three_d_geometry_set.

EXPRESS specification:

*)
TYPE composite_assembly_shape = SELECT
   (Advanced_brep_shape_representation, 
    Edge_based_wireframe_shape_representation, 
    External_geometric_model, 
    Faceted_brep_shape_representation, 
    Geometrically_bounded_surface_shape_representation, 
    Geometrically_bounded_wireframe_shape_representation, 
    Manifold_surface_shape_representation, 
    Shell_based_wireframe_shape_representation, 
    Tessellated_shape_representation, 
    Three_d_geometry_set);
END_TYPE; 
(*

## 4.2.3 core_shape

The core_shape type allows for the designation of the data types Advanced_brep_shape_representation, Beveled_sheet_representation, External_geometric_model, Faceted_brep_shape_representation, and Tessellated_shape_representation.

EXPRESS specification:

*)
TYPE core_shape = SELECT
   (Advanced_brep_shape_representation, 
    Beveled_sheet_representation, 
    External_geometric_model, 
    Faceted_brep_shape_representation, 
    Tessellated_shape_representation);
END_TYPE; 
(*

## 4.2.4 curve_or_point

The curve_or_point type allows for the designation of the data types Curve, and Point.

EXPRESS specification:

*)
TYPE curve_or_point = SELECT
   (Curve, 
    Point);
END_TYPE; 
(*

## 4.2.5 geometric_set_select

The geometric_set_select type allows for the designation of the data types Curve, Point, and Surface.

EXPRESS specification:

*)
TYPE geometric_set_select = SELECT
   (Curve, 
    Point, 
    Surface);
END_TYPE; 
(*

## 4.2.6 ply_laminate_shape

The ply_laminate_shape type is an extensible list of alternate data types that allows for the designation of the data typesAdvanced_brep_shape_representation, External_geometric_model, Composite_sheet_representation, Csg_model, Curve_swept_solid, Elementary_brep_shape_representation, Faceted_brep_shape_representation, Tessellated_shape_representation, and Three_d_geometry_set.

NOTE   The list of entity data types may be extended in application modules that use the constructs of this module.

EXPRESS specification:

*)
TYPE ply_laminate_shape = EXTENSIBLE GENERIC_ENTITY SELECT
   (Advanced_brep_shape_representation, 
    External_geometric_model, 
    Composite_sheet_representation, 
    Csg_model, 
    Curve_swept_solid, 
    Elementary_brep_shape_representation, 
    Faceted_brep_shape_representation, 
    Tessellated_shape_representation, 
    Three_d_geometry_set);
END_TYPE; 
(*

## 4.2.7 ply_piece_shape

The ply_piece_shape type allows for the designation of the data type Ply_shape.

EXPRESS specification:

*)
TYPE ply_piece_shape = SELECT
   (Ply_shape);
END_TYPE; 
(*

## 4.2.8 projection_method

A projection_method lists the shape modelling methods used to project the three-dimensional ply shape onto a plane or surface.

EXPRESS specification:

*)
TYPE projection_method = ENUMERATION OF 
   (projection_reference,
    surface_normal);
END_TYPE; 
(*

Enumerated item definitions:

projection_reference: the projection method is based on a projection reference.

surface_normal: the projection method is based on a surface normal which is a vector that is perpendicular to the tangent plane to this surface at a specific point.

## 4.3 ARM entity definitions

This subclause specifies the ARM entities for this module. Each ARM application entity is an atomic element that embodies a unique application concept and contains attributes specifying the data elements of the entity. The ARM entities and definitions are specified below.

## 4.3.1 Beveled_sheet_representation

A Beveled_sheet_representation is a type of Constituent_shape_representation that has a significant thickness, and whose edge is bevelled at a constant angle around its entire perimeter. It is an extension of the concept of a thin sheet to products that have significant thickness but can be formed to mate with a curved surface.

EXAMPLE    A honeycomb is a type of bevelled sheet representation.

EXPRESS specification:

*)
ENTITY Beveled_sheet_representation
  SUBTYPE OF (Constituent_shape_representation);
  bevel_angle : Angle_data_element;
  boundary_surface : Composite_sheet_representation;
  vertical_profile_height : Length_data_element;
END_ENTITY;
(*

Attribute definitions:

bevel_angle: specifies the angle from the plane normal to the defining surface of part's profile to the bevelled surface.

boundary_surface: specifies the base exterior boundary and defining surface of the part.

vertical_profile_height: specifies the vertical height of the part's profile from the base boundary to where the bevel begins.

## 4.3.2 Boundary_curve_set

A Boundary_curve_set is a type of Geometric_curve_set that is a set of curves that define the sheet edge. These curves define a closed area but are not necessarily restrained topologically. The curves that encompass the other curves are the outer boundary, while curves that define a closed area within the outer boundary are cut-outs.

EXPRESS specification:

*)
ENTITY Boundary_curve_set
  SUBTYPE OF (Geometric_curve_set);
  SELF\Geometric_set.elements RENAMED bounds : SET[1:?] OF Curve;
END_ENTITY;
(*

Attribute definitions:

bounds: an inherited attribute that is constrained to be set of Curves only and that defines the boundary aspect.

## 4.3.3 Composite_sheet_representation

A Composite_sheet_representation is a type of Constituent_shape_representation that defines a sheet product. A sheet product is a product whose thickness is constant and small in comparison to its length and width. Most sheet products will conform to the surface on which they are placed. The shape of a sheet product can be represented in a 3-dimensional model of the actual shape, as a shape projected onto the layup surface, or as a shape projected into a 2-dimensional plane.

EXAMPLE    A typical composite sheet is 0.1 to 0.4 millimetres thick while its length and width are much greater than 25 millimetres.

A Composite_sheet_representation may be one of the following:

- Face_based_sheet_representation,

- Geometric_sheet_representation.

EXPRESS specification:

*)
ENTITY Composite_sheet_representation
  SUPERTYPE OF (ONEOF (Face_based_sheet_representation,
                       Geometric_sheet_representation))
  SUBTYPE OF (Constituent_shape_representation);
END_ENTITY;
(*

## 4.3.4 Constituent_shape_representation

A Constituent_shape_representation is a type of Geometric_model that defines the shape of composite part constituents. A Constituent_shape_representation is one of the following:

- Three_d_geometry_set,

- Composite_sheet_representation,

- Beveled_sheet_representation.

EXPRESS specification:

*)
ENTITY Constituent_shape_representation
  ABSTRACT SUPERTYPE OF (ONEOF (Three_d_geometry_set,
                                Composite_sheet_representation,
                                Beveled_sheet_representation))
  SUBTYPE OF (Geometric_model);
END_ENTITY;
(*

## 4.3.5 Face_based_sheet_representation

A Face_based_sheet_representation is a type of Composite_sheet_representation that is defined topologically.

EXPRESS specification:

*)
ENTITY Face_based_sheet_representation
  SUBTYPE OF (Composite_sheet_representation);
  face_surface : Surface;
  inner_bounds : OPTIONAL SET[1:?] OF Loop;
  outer_bound : Loop;
END_ENTITY;
(*

Attribute definitions:

face_surface: the basis surface for the face.

inner_bounds: a set of topologically defined loops that define any cut-outs in the sheet. The value of this attribute need not be specified.

outer_bound: the topologically defined loop that is the outer edge of the sheet.

## 4.3.6 Flat_pattern_ply_shape

A Flat_pattern_ply_shape is a type of Ply_shape that defines the edge boundary of a ply in a plane. When the shape is laid up in the assembly, it will have the proper size to define the desired ply shape.

EXPRESS specification:

*)
ENTITY Flat_pattern_ply_shape
  SUBTYPE OF (Ply_shape);
  wrapup_origin_on_flat_pattern : Cartesian_point;
  wrapup_origin_on_surface : Cartesian_point;
END_ENTITY;
(*

Attribute definitions:

wrapup_origin_on_flat_pattern: the point of the flat pattern plane that corresponds directly to the wrapup_origin_on_surface.

wrapup_origin_on_surface: the point on the basis surface that corresponds directly to the wrapup_origin_on_flat_pattern.

## 4.3.7 Geometric_curve_set

A Geometric_curve_set is a type of Geometric_set which is a collection of two- or three-dimensional Points and Curves.

EXPRESS specification:

*)
ENTITY Geometric_curve_set
  SUBTYPE OF (Geometric_set);
  SELF\Geometric_set.elements : SET[1:?] OF curve_or_point;
END_ENTITY;
(*

Attribute definitions:

elements: an inherited attribute that is constrained to exclude Surfaces.

## 4.3.8 Geometric_set

A Geometric_set is a type of Detailed_geometric_model_element which is intended for the transfer of models when a topological structure is not available.

EXPRESS specification:

*)
ENTITY Geometric_set
  SUBTYPE OF (Detailed_geometric_model_element);
  elements : SET[1:?] OF geometric_set_select;
END_ENTITY;
(*

Attribute definitions:

elements: the geometric elements which make up the geometric_set, these may be Points, Curves or Surfaces; but are required to be of the same coordinate space dimensionality.

## 4.3.9 Geometric_sheet_representation

A Geometric_sheet_representation is a type of Composite_sheet_representation that is defined by a surface and a set of curves. These curves are related in the application to that surface. The curves define a closed area but are not necessarily constrained topologically.

EXPRESS specification:

*)
ENTITY Geometric_sheet_representation
  SUBTYPE OF (Composite_sheet_representation);
  basis_surface : Surface;
  cutouts : OPTIONAL SET[1:?] OF Boundary_curve_set;
  outer_edge : Boundary_curve_set;
END_ENTITY;
(*

Attribute definitions:

basis_surface: the Surface used to define the shape and having some known context to the part being represented. The surface may be the layup surface, inner-mould-line surface, outer-mould-line surface, etc.

cutouts: the cutouts specifies the sets of curves that define the areas to be removed from the interior of the part. The value of this attribute need not be specified.

outer_edge: the set of curves that define the outside edge (or boundary) of the part.

## 4.3.10 Laid_ply_shape

A Laid_ply_shape is a type of Ply_shape that defines the shape of the ply in the actual Ply_laminate. This is the physical shape that will stair up and down over the build-up of previously laid plies.

EXPRESS specification:

*)
ENTITY Laid_ply_shape
  SUBTYPE OF (Ply_shape);
END_ENTITY;
(*

## 4.3.11 Ply_shape

A Ply_shape is the association of a geometric representation to define the shape of a Ply.

NOTE    A Ply_shape may be a shape modified by the manufacturing process to provide material excess for functions such as trimming and tooling. In this case, the string 'manufacturing' is prepended to the specification of the type of Ply_shape in the mapping specification mapping rules.

NOTE    Each type of Ply_shape may be generated from or related to, another Ply_shape.

EXAMPLE    A Flat_pattern_ply_shape may be generated from a Laid_ply_shape, and a View_ply_shape can be projected to the surface and create a Surface_ply_shape.

A Ply_shape may be one of the following:

- Projected_ply_shape,

- Laid_ply_shape,

- Flat_pattern_ply_shape.

EXPRESS specification:

*)
ENTITY Ply_shape
  SUPERTYPE OF (ONEOF (Projected_ply_shape,
                       Laid_ply_shape,
                       Flat_pattern_ply_shape));
  defining_model : ply_laminate_shape;
  basis : OPTIONAL Ply_shape;
END_ENTITY;
(*

Attribute definitions:

defining_model: the geometric representation used for the Ply_shape.

basis: the shape representation that the Ply_shape is derived from or that is used as its basis of definition. The value of this attribute need not be specified.

## 4.3.12 Projected_ply_shape

A Projected_ply_shape is a type of Ply_shape that has its shape modelled by a projection on a surface or a plane. The surface has some defined context for the Ply.

NOTE    A Projected_ply_shape is a projection of the actual ply. Projected shapes do not represent the build-up of the surface, but all lie on the same surface or plane. These shapes are defined during the ply layout in design.

A Projected_ply_shape may be one of the following:

- View_ply_shape,

- Surface_ply_shape.

EXPRESS specification:

*)
ENTITY Projected_ply_shape
  SUPERTYPE OF (ONEOF (View_ply_shape,
                       Surface_ply_shape))
  SUBTYPE OF (Ply_shape);
  method : projection_method;
  projection_direction : OPTIONAL Direction;
END_ENTITY;
(*

Attribute definitions:

method: the reference direction used in projecting the shape on the surface. The value of the method shall be one of the following:

- projection_reference;

- surface_normal.

projection_direction: the direction from the given representation to the actual Laid_ply_shape. The value of this attribute need not be specified.

## 4.3.13 Surface_ply_shape

A Surface_ply_shape is a type of Projected_ply_shape that is defined on a surface defining some aspect of a composite part. The surface may be the layup surface, the outer mould line (OML), or the inner mould line (IML).

EXPRESS specification:

*)
ENTITY Surface_ply_shape
  SUBTYPE OF (Projected_ply_shape);
  offset : Length_data_element;
  surface_role : STRING;
END_ENTITY;
(*

Attribute definitions:

offset: the distance from the modelled surface to the layup surface of the part. The offset distance will be zero if the modelled surface is the layup surface.

surface_role: the context of the surface defined to the actual part.

NOTE    surface role should be limited to text defining contexts.

EXAMPLE    'layup surface', 'outer mould line', and 'inner mould line' are defining the context for the surface role.

## 4.3.14 Three_d_geometry_set

A Three_d_geometry_set is a type of Constituent_shape_representation and a type of Geometrically_bounded_surface_shape_representation that defines 3D boundary curve and surface geometry without topology. The surfaces used in the representation have a defined model surface context.

EXPRESS specification:

*)
ENTITY Three_d_geometry_set
  SUBTYPE OF (Constituent_shape_representation, Geometrically_bounded_surface_shape_representation);
  basis_role : STRING;
  basis_surface : Surface;
  defining_boundary : SET[1:?] OF Curve;
END_ENTITY;
(*

Attribute definitions:

basis_role: the context of the basis surface for the part.

EXAMPLE    Specific context is 'layup surface', 'outer mould line', and 'inner mould line'.

basis_surface: the Surface used to define the part shape. The context of the surface is defined in the attribute basis_role.

defining_boundary: the set of Curve objects that define the edge aspect of a part.

## 4.3.15 View_ply_shape

A View_ply_shape is a type of Projected_ply_shape in which the projection is to a plane. The projection direction establishes the relationship of the plane to the actual part.

EXPRESS specification:

*)
ENTITY View_ply_shape
  SUBTYPE OF (Projected_ply_shape);
END_ENTITY;
(*

*)
END_SCHEMA;  -- Composite_constituent_shape_arm
(*

© ISO 2019 — All rights reserved