## 5.2 MIM EXPRESS short listing

This clause specifies the EXPRESS schema derived from the mapping table. It uses elements from the common resources or from other application modules and defines the EXPRESS constructs that are specific to this part of ISO 10303.

This clause constitutes the Module Interpreted Module (MIM) of the application module.

This clause also specifies the modifications that apply to the constructs imported from the common resources.

The following restrictions apply to the use, in this schema, of constructs defined in common resources or in application modules:

- use of a supertype entity does not make applicable any of its specializations, unless the specialization is also imported in the MIM schema;

- use of a SELECT type does not make applicable any of its listed types unless the listed type is also imported in the MIM schema.

Short names of entities defined in this schema are described in Annex A. Unambiguous identification of this schema is defined in Annex B.

EXPRESS specification: 

*)
SCHEMA Part_and_zone_laminate_tables_mim;

USE FROM Composite_constituent_shape_mim;    --  ISO/TS 10303-1767

USE FROM Ply_orientation_specification_mim;    --  ISO/TS 10303-1772

USE FROM product_definition_schema    --  ISO 10303-41
  (product_related_product_category);

USE FROM product_structure_schema    --  ISO 10303-44
  (next_assembly_usage_occurrence);

USE FROM Shape_property_assignment_mim;    --  ISO/TS 10303-1032

USE FROM Stock_material_mim;    --  ISO/TS 10303-1771
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Composite_constituent_shape_mim | ISO/TS 10303-1767 |
| --- | --- |
| Ply_orientation_specification_mim | ISO/TS 10303-1772 |
| product_definition_schema | ISO 10303-41 |
| product_structure_schema | ISO 10303-44 |
| Shape_property_assignment_mim | ISO/TS 10303-1032 |
| Stock_material_mim | ISO/TS 10303-1771 |

NOTE 2   See Annex D, Figures D.1and D.2 for a graphical representation of this schema.

## 5.2.1 MIM entity definitions

This subclause specifies the MIM entities for this module. The MIM entities and definitions are specified below.

## 5.2.1.1 composite_assembly_sequence_definition

A composite_assembly_sequence_definition is a type of product_definition that implements the ARM concept of Composite_assembly_sequence_definition.

EXPRESS specification:

*)
ENTITY composite_assembly_sequence_definition
  SUBTYPE OF (product_definition);
WHERE
  WR1: SIZEOF (QUERY (pdr <* USEDIN (SELF, 'PRODUCT_DEFINITION_SCHEMA.' + 'PRODUCT_DEFINITION_RELATIONSHIP.' + 'RELATING_PRODUCT_DEFINITION') | 'PRODUCT_STRUCTURE_SCHEMA.' + 'NEXT_ASSEMBLY_USAGE_OCCURRENCE' IN TYPEOF (pdr))) > 0;
END_ENTITY;
(*

Formal propositions:

WR1: A composite_assembly_sequence_definition shall be used by one or many next_assembly_usage_occurrence.

## 5.2.1.2 composite_assembly_table

A composite_assembly_table is a type of part_laminate_table that implements the ARM concept of Composite_assembly_table.

EXPRESS specification:

*)
ENTITY composite_assembly_table
  SUBTYPE OF (part_laminate_table);
END_ENTITY;
(*

## 5.2.1.3 laminate_table

A laminate_table is a type of product_definition that implements the ARM concept of Laminate_table. A laminate_table may be a part_laminate_table or azone_structural_makeup.

EXPRESS specification:

*)
ENTITY laminate_table
  SUPERTYPE OF (ONEOF (part_laminate_table,
                       zone_structural_makeup))
  SUBTYPE OF (product_definition);
END_ENTITY;
(*

## 5.2.1.4 part_laminate_table

A part_laminate_table is a type of laminate_table that implements the ARM concept of Part_laminate_table. A part_laminate_table may be aply_laminate_table or a composite_assembly_table.

EXPRESS specification:

*)
ENTITY part_laminate_table
  SUPERTYPE OF (ONEOF (composite_assembly_table,
                       ply_laminate_table))
  SUBTYPE OF (laminate_table);
END_ENTITY;
(*

## 5.2.1.5 percentage_laminate_table

A percentage_laminate_table is a type of zone_structural_makeup that implements the ARM concept of Percentage_laminate_table.

EXPRESS specification:

*)
ENTITY percentage_laminate_table
  SUBTYPE OF (zone_structural_makeup);
END_ENTITY;
(*

## 5.2.1.6 ply_laminate_sequence_definition

A ply_laminate_sequence_definition is a type of product_definition that implements the ARM concept of Ply_laminate_sequence_definition.

EXPRESS specification:

*)
ENTITY ply_laminate_sequence_definition
  SUBTYPE OF (product_definition);
WHERE
  WR1: SIZEOF (QUERY (pdr <* USEDIN (SELF, 'PRODUCT_DEFINITION_SCHEMA.' + 'PRODUCT_DEFINITION_RELATIONSHIP.' + 'RELATING_PRODUCT_DEFINITION') | 'PRODUCT_STRUCTURE_SCHEMA.' + 'NEXT_ASSEMBLY_USAGE_OCCURRENCE' IN TYPEOF (pdr))) > 0;
END_ENTITY;
(*

Formal propositions:

WR1: A ply_laminate_sequence_definition shall be used by one or many next_assembly_usage_occurrence.

NOTE    The next_assembly_usage_occurrence.relating_product_definition shall be the ply_laminate_sequence_definition. Thenext_assembly_usage_occurrence.related_product_definition thus associated comprise the SET of product_definition entities for the table.

## 5.2.1.7 ply_laminate_table

A ply_laminate_table is a type of part_laminate_table that implements the ARM concept of Ply_laminate_table.

EXPRESS specification:

*)
ENTITY ply_laminate_table
  SUBTYPE OF (part_laminate_table);
END_ENTITY;
(*

## 5.2.1.8 smeared_material_definition

A smeared_material_definition is a type of zone_structural_makeup. A smeared_material_definition is an idealized product_definition of a laminate product that smears the constituent materials together into a single definition that is representative of the whole.

EXPRESS specification:

*)
ENTITY smeared_material_definition
  SUBTYPE OF (zone_structural_makeup);
END_ENTITY;
(*

## 5.2.1.9 thickness_laminate_table

A thickness_laminate_table is a type of zone_structural_makeup that implements the ARM concept of Thickness_laminate_table.

EXPRESS specification:

*)
ENTITY thickness_laminate_table
  SUBTYPE OF (zone_structural_makeup);
END_ENTITY;
(*

## 5.2.1.10 zone_structural_makeup

A zone_structural_makeup is a type of laminate_table that implements the ARM concept of Zone_structural_makeup. A zone_structural_makeup may be either a thickness_laminate_table and a smeared_material_definition or a percentage_laminate_table and a smeared_material_definition, or athickness_laminate_table, or a percentage_laminate_table, or a smeared_material_definition.

EXPRESS specification:

*)
ENTITY zone_structural_makeup
  SUPERTYPE OF (ONEOF ((thickness_laminate_table
               AND smeared_material_definition), (
                  percentage_laminate_table
              AND smeared_material_definition),
                 thickness_laminate_table,
                 percentage_laminate_table,
                 smeared_material_definition))
  SUBTYPE OF (laminate_table);
END_ENTITY;
(*

## 5.2.2 MIM subtype constraint definition

This subclause specifies the MIM subtype constraint for this module. The subtype constraint places a constraint on the possible super-type / subtype instantiations. The MIM subtype constraint and definition is specified below.

## 5.2.2.1 pos_product_definition_subtypes

The pos_product_definition_subtypes constraint specifies a constraint that applies to instances of subtypes of product_definition.

EXPRESS specification:

*)
SUBTYPE_CONSTRAINT pos_product_definition_subtypes FOR product_definition;
  ONEOF (composite_assembly_sequence_definition,
         laminate_table,
         ply_laminate_sequence_definition);
END_SUBTYPE_CONSTRAINT;
(*

## 5.2.3 MIM rule definitions

This subclause specifies the MIM rules for this module. The MIM rules and definitions are specified below.

## 5.2.3.1 component_class_for_assembly_select

The component_class_for_assembly_select rule states that products associated with a composite_assembly_sequence_definition shall have aproduct_related_product_category.name of "ply", "ply laminate", "filament laminate", "processed core", or "composite assembly".

EXPRESS specification:

*)
RULE component_class_for_assembly_select FOR 
(composite_assembly_sequence_definition, next_assembly_usage_occurrence, product_related_product_category);

LOCAL

i,j,k :  INTEGER :=0;

dkuhr :  LOGICAL :=TRUE;

nnauo :  INTEGER :=0;

nprpc :  INTEGER :=0;

rp    :  product;

END_LOCAL;

REPEAT i:= LOINDEX (composite_assembly_sequence_definition) TO

HIINDEX (composite_assembly_sequence_definition);

nnauo := 0;

REPEAT j:= LOINDEX (next_assembly_usage_occurrence) TO

HIINDEX (next_assembly_usage_occurrence);

IF (composite_assembly_sequence_definition[i] =

next_assembly_usage_occurrence[j].relating_product_definition) THEN

rp := next_assembly_usage_occurrence[j].related_product_definition.

formation.of_product;

nprpc := 0;

REPEAT k:= LOINDEX (product_related_product_category) TO

HIINDEX (product_related_product_category);

IF ((rp IN product_related_product_category[k].products) AND

(product_related_product_category[k].name IN ['ply',

'ply laminate', 'filament laminate', 'processed core',

'composite assembly'])) THEN

nprpc := nprpc + 1;

END_IF;

END_REPEAT;

IF (nprpc = 1) THEN

nnauo := nnauo + 1;

ELSE

dkuhr := FALSE;

ESCAPE;

END_IF;

END_IF;

END_REPEAT;

IF (dkuhr = FALSE) THEN

ESCAPE;

END_IF;

IF(nnauo = 0) THEN

dkuhr := FALSE;

ESCAPE;

END_IF;

END_REPEAT;

WHERE
  WR1: dkuhr;
END_RULE; 
(*

Argument definitions:

composite_assembly_sequence_definition : the set of all instances of composite_assembly_sequence_definition.

next_assembly_usage_occurrence : the set of all instances of next_assembly_usage_occurrence.

product_related_product_category : the set of all instances of product_related_product_category.

Formal propositions:

WR1: Composite_assembly_sequence_definitions shall be related through at least one next_assembly_usage_occurrence relationship to product_definitionswhich are associated with products that are in the product_related_product_category with the name atttribute of "ply", "ply laminate", "filament laminate", "composite assembly", or "processed core".

## 5.2.3.2 ply_reference

The ply_reference rules states that products associated with a ply_laminate_sequence_definition shall have a product_related_product_category.name of "ply".

EXPRESS specification:

*)
RULE ply_reference FOR 
(ply_laminate_sequence_definition, next_assembly_usage_occurrence, product_related_product_category);

LOCAL

i,j,k :  INTEGER;

dkuhr :  LOGICAL := TRUE;

nnauo :  INTEGER;

nprpc :  INTEGER := 0;

rp    :  product;

END_LOCAL;

REPEAT i:= LOINDEX (ply_laminate_sequence_definition) TO

HIINDEX (ply_laminate_sequence_definition);

nnauo := 0;

REPEAT j:= LOINDEX (next_assembly_usage_occurrence) TO

HIINDEX (next_assembly_usage_occurrence);

IF (ply_laminate_sequence_definition[i] =

next_assembly_usage_occurrence[j].relating_product_definition) THEN

rp := next_assembly_usage_occurrence[j].related_product_definition.

formation.of_product;

nprpc := 0;

REPEAT k:= LOINDEX (product_related_product_category) TO

HIINDEX (product_related_product_category);

IF ((product_related_product_category[k].name = 'ply') AND

(rp IN product_related_product_category[k].products)) THEN

nprpc := nprpc + 1;

END_IF;

END_REPEAT;

IF (nprpc = 1) THEN

nnauo := nnauo + 1;

ELSE

dkuhr := FALSE;

ESCAPE;

END_IF;

END_IF;

END_REPEAT;

IF (dkuhr = FALSE) THEN

ESCAPE;

END_IF;

IF(nnauo = 0) THEN

dkuhr := FALSE;

ESCAPE;

END_IF;

END_REPEAT;

WHERE
  WR1: dkuhr;
END_RULE; 
(*

Argument definitions:

ply_laminate_sequence_definition : the set of all instances of ply_laminate_sequence_definition.

next_assembly_usage_occurrence : the set of all instances of next_assembly_usage_occurrence.

product_related_product_category : the set of all instances of product_related_product_category.

Formal propositions:

WR1: Ply_laminate_sequence_definitions shall be related through at least one next_assembly_usage_occurrence relationship to product_definitions which are associated with products that are in the product_related_product_category with the name atttribute of "ply".

*)
END_SCHEMA;  -- Part_and_zone_laminate_tables_mim
(*

© ISO 2019 — All rights reserved