## 4.1 Required AM ARMs

The following EXPRESS interface statements specify the elements imported from the ARMs of other application modules.

EXPRESS specification:

*)
USE FROM Part_view_definition_arm;    --  ISO/TS 10303-1023

USE FROM Shape_feature_arm;    --  ISO/TS 10303-1764

USE FROM Shape_property_assignment_arm;    --  ISO/TS 10303-1032
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Part_view_definition_arm | ISO/TS 10303-1023 |
| --- | --- |
| Shape_feature_arm | ISO/TS 10303-1764 |
| Shape_property_assignment_arm | ISO/TS 10303-1032 |

NOTE 2   See Annex C, Figures C.1and C.2 for a graphical representation of this schema.

## 4.2 ARM type definitions

This subclause specifies the ARM types for this application module. The ARM types and definitions are specified below.

## 4.2.1 part_shape_element_definition_select

The part_shape_element_definition_select type is an extensible list of alternate data types that allow for the designation of the data types Shape_feature_definition, and Part_shape_element.

NOTE   The list of entity data types may be extended in application modules that use the constructs of this module.

EXPRESS specification:

*)
TYPE part_shape_element_definition_select = EXTENSIBLE GENERIC_ENTITY SELECT
   (Shape_feature_definition, 
    Part_shape_element);
END_TYPE; 
(*

## 4.2.2 part_view_definition_or_part_shape_element_select

The part_view_definition_or_part_shape_element_select type is an extensible list of alternate data types that allows for the designation of the data types Part_shape_element, and Part_view_definition.

NOTE   The list of entity data types may be extended in application modules that use the constructs of this module.

EXPRESS specification:

*)
TYPE part_view_definition_or_part_shape_element_select = EXTENSIBLE SELECT
   (Part_shape_element, 
    Part_view_definition);
END_TYPE; 
(*

## 4.2.3 ps_additional_application_domain_enumeration

A ps_additional_application_domain_enumeration provides predefined identifiers for the additional application domains of Additional_view_definition_contexts.

EXPRESS specification:

*)
TYPE ps_additional_application_domain_enumeration = ENUMERATION BASED_ON additional_application_domain_enumeration WITH 
   (electrical,
    thermal);
END_TYPE; 
(*

Enumerated item definitions:

electrical: the application domain is electrical.

thermal: the application domain is thermal.

## 4.2.4 ps_initial_application_domain_enumeration

The ps_initial_application_domain_enumeration provides predefined identifiers for the initial application domains of Product_view_definitions.

EXPRESS specification:

*)
TYPE ps_initial_application_domain_enumeration = ENUMERATION BASED_ON initial_application_domain_enumeration WITH 
   (electrical,
    thermal);
END_TYPE; 
(*

Enumerated item definitions:

electrical: the application domain is electrical.

thermal: the application domain is thermal.

## 4.3 ARM entity definitions

This subclause specifies the ARM entities for this module. Each ARM application entity is an atomic element that embodies a unique application concept and contains attributes specifying the data elements of the entity. The ARM entities and definitions are specified below.

## 4.3.1 General_part_feature

A General_part_feature is a type of Part_shape_element that represents an external shape feature of a Part_view_definition.

EXPRESS specification:

*)
ENTITY General_part_feature
  SUBTYPE OF (Part_shape_element);
DERIVE
  SELF\Shape_element.product_definitional : BOOLEAN := TRUE;
END_ENTITY;
(*

Attribute definitions:

product_definitional: the General_part_feature shall be definitional.

## 4.3.2 Part_shape_element

A Part_shape_element is a type of Shape_element that represents an element of the shape of a Part_view_definition or another Part_shape_element.

EXPRESS specification:

*)
ENTITY Part_shape_element
  SUBTYPE OF (Shape_element);
  SELF\Shape_element.associated_definition : part_view_definition_or_part_shape_element_select;
  definition : OPTIONAL part_shape_element_definition_select;
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute whose value shall be of type Part_view_definition.

definition: specifies a Shape_feature_definition or another Part_shape_element that is the definition of this Part_shape_element. The value of this attribute need not be specified.

## 4.3.3 Part_shape_element_relationship

A Part_shape_element_relationship is a type of Shape_element_relationship that represents is a relationship between two instances of Part_shape_element.

EXPRESS specification:

*)
ENTITY Part_shape_element_relationship
  SUBTYPE OF (Shape_element_relationship);
  SELF\Shape_element_relationship.related : Part_shape_element;
  SELF\Shape_element_relationship.relating : Part_shape_element;
END_ENTITY;
(*

Attribute definitions:

related: an inherited attribute whose value shall be of type Part_shape_element.

relating: an inherited attribute whose value shall be of type Part_shape_element.

*)
END_SCHEMA;  -- Part_shape_arm
(*

© ISO 2019 — All rights reserved