## 5.2 MIM EXPRESS short listing

This clause specifies the EXPRESS schema derived from the mapping table. It uses elements from the common resources or from other application modules and defines the EXPRESS constructs that are specific to this part of ISO 10303.

This clause constitutes the Module Interpreted Module (MIM) of the application module.

This clause also specifies the modifications that apply to the constructs imported from the common resources.

The following restrictions apply to the use, in this schema, of constructs defined in common resources or in application modules:

- use of a supertype entity does not make applicable any of its specializations, unless the specialization is also imported in the MIM schema;

- use of a SELECT type does not make applicable any of its listed types unless the listed type is also imported in the MIM schema.

Short names of entities defined in this schema are described in Annex A. Unambiguous identification of this schema is defined in Annex B.

EXPRESS specification: 

*)
SCHEMA Assembly_functional_interface_requirement_mim;

USE FROM Altered_part_mim;    --  ISO/TS 10303-1602

USE FROM Functional_specification_mim;    --  ISO/TS 10303-1679
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Altered_part_mim | ISO/TS 10303-1602 |
| --- | --- |
| Functional_specification_mim | ISO/TS 10303-1679 |

NOTE 2   See Annex D, Figures D.1and D.2 for a graphical representation of this schema.

## 5.2.1 MIM type definition

This subclause specifies the MIM type for this application module. The MIM type and definition is specified below.

## 5.2.1.1 afir_document_reference_item

The afir_document_reference_item type is an extension of the document_reference_item type. It adds the data type protocol_physical_layer_definition to the list of alternate data types.

EXPRESS specification:

*)
TYPE afir_document_reference_item = SELECT BASED_ON document_reference_item WITH 
   (protocol_physical_layer_definition);
END_TYPE; 
(*

## 5.2.2 MIM entity definitions

This subclause specifies the MIM entities for this module. The MIM entities and definitions are specified below.

## 5.2.2.1 minimally_defined_connector

A minimally_defined_connector is a type of packaged_part that implements the ARM concept of Minimally_defined_connector.

EXPRESS specification:

*)
ENTITY minimally_defined_connector
  SUBTYPE OF (packaged_part);
END_ENTITY;
(*

## 5.2.2.2 protocol_physical_layer_definition

A protocol_physical_layer_definition is a type of product_definition that implements the ARM concept of Protocol_physical_layer_definition.

EXPRESS specification:

*)
ENTITY protocol_physical_layer_definition
  SUBTYPE OF (product_definition);
END_ENTITY;
(*

## 5.2.2.3 protocol_physical_layer_definition_with_characterization

A protocol_physical_layer_definition_with_characterization is a type of protocol_physical_layer_definition that implements the ARM concept ofProtocol_physical_layer_definition_with_characterization.

EXPRESS specification:

*)
ENTITY protocol_physical_layer_definition_with_characterization
  SUBTYPE OF (protocol_physical_layer_definition);
END_ENTITY;
(*

*)
END_SCHEMA;  -- Assembly_functional_interface_requirement_mim
(*

© ISO 2019 — All rights reserved