## 5.2 MIM EXPRESS short listing

This clause specifies the EXPRESS schema derived from the mapping table. It uses elements from the common resources or from other application modules and defines the EXPRESS constructs that are specific to this part of ISO 10303.

This clause constitutes the Module Interpreted Module (MIM) of the application module.

This clause also specifies the modifications that apply to the constructs imported from the common resources.

The following restrictions apply to the use, in this schema, of constructs defined in common resources or in application modules:

- use of a supertype entity does not make applicable any of its specializations, unless the specialization is also imported in the MIM schema;

- use of a SELECT type does not make applicable any of its listed types unless the listed type is also imported in the MIM schema.

Short names of entities defined in this schema are described in Annex A. Unambiguous identification of this schema is defined in Annex B.

EXPRESS specification: 

*)
SCHEMA Assembly_physical_interface_requirement_mim;

USE FROM Assembly_functional_interface_requirement_mim;    --  ISO/TS 10303-1635

USE FROM Layered_3d_shape_mim;    --  ISO/TS 10303-1694

USE FROM Printed_physical_layout_template_mim;    --  ISO/TS 10303-1737

USE FROM product_structure_schema    --  ISO 10303-44
  (specified_higher_usage_occurrence); 
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Assembly_functional_interface_requirement_mim | ISO/TS 10303-1635 |
| --- | --- |
| Layered_3d_shape_mim | ISO/TS 10303-1694 |
| Printed_physical_layout_template_mim | ISO/TS 10303-1737 |
| product_structure_schema | ISO 10303-44 |

NOTE 2   See Annex D, Figures D.1and D.2 for a graphical representation of this schema.

## 5.2.1 MIM type definitions

This subclause specifies the MIM types for this application module. The MIM types and definitions are specified below.

## 5.2.1.1 apir_document_reference_item

The apir_document_reference_item type is an extension of the document_reference_item type. It adds the data type protocol_physical_layer_definition to the list of alternate data types.

NOTE  The list of entity data types may be extended in application modules that use the constructs of this module.

EXPRESS specification:

*)
TYPE apir_document_reference_item = EXTENSIBLE GENERIC_ENTITY SELECT BASED_ON document_reference_item WITH 
   (protocol_physical_layer_definition);
END_TYPE; 
(*

## 5.2.1.2 apir_requirement_assigned_item

The apir_requirement_assigned_item type is an extension of the requirement_assigned_item type. It adds the data typespredefined_requirement_view_definition, and shape_aspect to the list of alternate data types.

NOTE  The list of entity data types may be extended in application modules that use the constructs of this module.

EXPRESS specification:

*)
TYPE apir_requirement_assigned_item = EXTENSIBLE GENERIC_ENTITY SELECT BASED_ON requirement_assigned_item WITH 
   (predefined_requirement_view_definition, 
    shape_aspect);
END_TYPE; 
(*

## 5.2.2 MIM entity definition

This subclause specifies the MIM entity for this module. The MIM entity and definition is specified below.

## 5.2.2.1 protocol_requirement_allocation_to_part_terminal

A protocol_requirement_allocation_to_part_terminal is a type of requirement_assignment that implements the ARM concept ofProtocol_requirement_allocation_to_part_terminal.

EXPRESS specification:

*)
ENTITY protocol_requirement_allocation_to_part_terminal
  SUBTYPE OF (requirement_assignment);
END_ENTITY;
(*

*)
END_SCHEMA;  -- Assembly_physical_interface_requirement_mim
(*

© ISO 2019 — All rights reserved