## 5.2 MIM EXPRESS short listing

This clause specifies the EXPRESS schema derived from the mapping table. It uses elements from the common resources or from other application modules and defines the EXPRESS constructs that are specific to this part of ISO 10303.

This clause constitutes the Module Interpreted Module (MIM) of the application module.

This clause also specifies the modifications that apply to the constructs imported from the common resources.

The following restrictions apply to the use, in this schema, of constructs defined in common resources or in application modules:

- use of a supertype entity does not make applicable any of its specializations, unless the specialization is also imported in the MIM schema;

- use of a SELECT type does not make applicable any of its listed types unless the listed type is also imported in the MIM schema.

Short names of entities defined in this schema are described in Annex A. Unambiguous identification of this schema is defined in Annex B.

EXPRESS specification: 

*)
SCHEMA Assembly_technology_mim;

USE FROM Feature_and_connection_zone_mim;    --  ISO/TS 10303-1671

USE FROM Generic_material_aspects_mim;    --  ISO/TS 10303-1681

USE FROM Physical_component_feature_mim;    --  ISO/TS 10303-1721

USE FROM Product_view_definition_reference_mim;    --  ISO/TS 10303-1823

USE FROM Requirement_assignment_mim;    --  ISO/TS 10303-1233

REFERENCE FROM product_property_definition_schema    --  ISO 10303-41
  (acyclic_shape_aspect_relationship); 
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Feature_and_connection_zone_mim | ISO/TS 10303-1671 |
| --- | --- |
| Generic_material_aspects_mim | ISO/TS 10303-1681 |
| Physical_component_feature_mim | ISO/TS 10303-1721 |
| Product_view_definition_reference_mim | ISO/TS 10303-1823 |
| Requirement_assignment_mim | ISO/TS 10303-1233 |
| product_property_definition_schema | ISO 10303-41 |

NOTE 2   See Annex D, Figures D.1and D.2 for a graphical representation of this schema.

## 5.2.1 MIM type definitions

This subclause specifies the MIM types for this application module. The MIM types and definitions are specified below.

## 5.2.1.1 at_external_identification_item

The at_external_identification_item type is an extension of the external_identification_item type. It adds the data type assembly_bond_definition to the list of alternate data types.

EXPRESS specification:

*)
TYPE at_external_identification_item = SELECT BASED_ON external_identification_item WITH 
   (assembly_bond_definition);
END_TYPE; 
(*

## 5.2.1.2 at_requirement_assigned_item

The at_requirement_assigned_item type is an extension of the requirement_assigned_item type. It adds the data type assembly_joint to the list of alternate data types.

EXPRESS specification:

*)
TYPE at_requirement_assigned_item = SELECT BASED_ON requirement_assigned_item WITH 
   (assembly_joint);
END_TYPE; 
(*

## 5.2.2 MIM entity definitions

This subclause specifies the MIM entities for this module. The MIM entities and definitions are specified below.

## 5.2.2.1 assembly_bond_definition

An assembly_bond_definition is a type of shape_aspect that implements the ARM concept of Assembly_bond_definition. An assembly_bond_definitiondefines a joint that uses material to bond together portions of the assembly.

EXPRESS specification:

*)
ENTITY assembly_bond_definition
  SUBTYPE OF (shape_aspect);
UNIQUE
  UR1: SELF\shape_aspect.name;
END_ENTITY;
(*

Formal propositions:

UR1: The name shall be unique within a population of assembly_bond_definition.

## 5.2.2.2 assembly_joint

An assembly_joint is a type of component_feature_joint that implements the ARM concepts of:

- Assembly_joint;

- Assembled_with_bonding;

- Assembled_with_fasteners.

An assembly_joint is the result of a physical connection between two components in the assembly. The order shall be inferred from the use of relating and related attributes on shape_aspect_relationship.

EXPRESS specification:

*)
ENTITY assembly_joint
  SUBTYPE OF (component_feature_joint);
WHERE
  WR1: acyclic_shape_aspect_relationship(SELF, [SELF\shape_aspect_relationship.related_shape_aspect], 'ASSEMBLY_TECHNOLOGY_MIM.'+ 'ASSEMBLY_JOINT');
END_ENTITY;
(*

Formal propositions:

WR1: The assembly_joint shall have a relating_shape_aspect that references either a component_terminal with a description of 'assembly module component terminal', 'bare die component terminal', 'interconnect component join terminal', 'interconnect module component terminal', 'package terminal occurrence', or 'packaged component join terminal', or a physical_component_interface_terminal or a component_mounting_feature or aninterconnect_module_component_surface_feature.

## 5.2.2.3 component_mating_constraint_condition

A component_mating_constraint_condition is a type of shape_aspect that implements the ARM concept of Component_mating_constraint_condition.

EXPRESS specification:

*)
ENTITY component_mating_constraint_condition
  SUBTYPE OF (shape_aspect);
UNIQUE
  UR1: SELF\shape_aspect.name, SELF\shape_aspect.of_shape;
END_ENTITY;
(*

Formal propositions:

UR1: The combination of name and of_shape shall be unique within a population of component_mating_constraint_condition.

## 5.2.2.4 connection_zone_based_assembly_joint

A connection_zone_based_assembly_joint is a type of assembly_joint that implements the ARM concept of Connection_zone_based_assembly_joint. A connection_zone_based_assembly_joint includes the identification of the connection zones that are to be mated in addition to the specific features that are to be mated.

EXPRESS specification:

*)
ENTITY connection_zone_based_assembly_joint
  SUBTYPE OF (assembly_joint);
END_ENTITY;
(*

*)
END_SCHEMA;  -- Assembly_technology_mim
(*

© ISO 2019 — All rights reserved