## 5.2 MIM EXPRESS short listing

This clause specifies the EXPRESS schema derived from the mapping table. It uses elements from the common resources or from other application modules and defines the EXPRESS constructs that are specific to this part of ISO 10303.

This clause constitutes the Module Interpreted Module (MIM) of the application module.

This clause also specifies the modifications that apply to the constructs imported from the common resources.

The following restrictions apply to the use, in this schema, of constructs defined in common resources or in application modules:

- use of a supertype entity does not make applicable any of its specializations, unless the specialization is also imported in the MIM schema;

- use of a SELECT type does not make applicable any of its listed types unless the listed type is also imported in the MIM schema.

Short names of entities defined in this schema are described in Annex A. Unambiguous identification of this schema is defined in Annex B.

EXPRESS specification: 

*)
SCHEMA Composite_constituent_shape_mim;

USE FROM Advanced_boundary_representation_mim;    --  ISO/TS 10303-1514

USE FROM Construction_geometry_mim;    --  ISO/TS 10303-1131

USE FROM Constructive_solid_geometry_3d_mim;    --  ISO/TS 10303-1068

USE FROM Curve_swept_solid_mim;    --  ISO/TS 10303-1659

USE FROM Edge_based_wireframe_mim;    --  ISO/TS 10303-1501

USE FROM Elementary_boundary_representation_mim;    --  ISO/TS 10303-1329

USE FROM External_model_mim;    --  ISO/TS 10303-1033

USE FROM Faceted_boundary_representation_mim;    --  ISO/TS 10303-1512

USE FROM geometric_model_schema    --  ISO 10303-42
  (geometric_curve_set, 
   geometric_set);

USE FROM Geometrically_bounded_surface_mim;    --  ISO/TS 10303-1507

USE FROM Geometrically_bounded_wireframe_mim;    --  ISO/TS 10303-1510

USE FROM Manifold_surface_mim;    --  ISO/TS 10303-1509

USE FROM product_property_representation_schema    --  ISO 10303-41
  (shape_representation_relationship);

USE FROM Shell_based_wireframe_mim;    --  ISO/TS 10303-1502

USE FROM Tessellated_geometry_mim;    --  ISO/TS 10303-1819

USE FROM Value_with_unit_extension_mim;    --  ISO/TS 10303-1753
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Advanced_boundary_representation_mim | ISO/TS 10303-1514 |
| --- | --- |
| Construction_geometry_mim | ISO/TS 10303-1131 |
| Constructive_solid_geometry_3d_mim | ISO/TS 10303-1068 |
| Curve_swept_solid_mim | ISO/TS 10303-1659 |
| Edge_based_wireframe_mim | ISO/TS 10303-1501 |
| Elementary_boundary_representation_mim | ISO/TS 10303-1329 |
| External_model_mim | ISO/TS 10303-1033 |
| Faceted_boundary_representation_mim | ISO/TS 10303-1512 |
| geometric_model_schema | ISO 10303-42 |
| Geometrically_bounded_surface_mim | ISO/TS 10303-1507 |
| Geometrically_bounded_wireframe_mim | ISO/TS 10303-1510 |
| Manifold_surface_mim | ISO/TS 10303-1509 |
| product_property_representation_schema | ISO 10303-41 |
| Shell_based_wireframe_mim | ISO/TS 10303-1502 |
| Tessellated_geometry_mim | ISO/TS 10303-1819 |
| Value_with_unit_extension_mim | ISO/TS 10303-1753 |

NOTE 2   See Annex D, Figures D.1and D.2 for a graphical representation of this schema.

## 5.2.1 MIM entity definitions

This subclause specifies the MIM entities for this module. The MIM entities and definitions are specified below.

## 5.2.1.1 beveled_sheet_representation

A beveled_sheet_representation is a type of shape_representation that implements the ARM concept of Beveled_sheet_representation.

EXPRESS specification:

*)
ENTITY beveled_sheet_representation
  SUBTYPE OF (shape_representation);
END_ENTITY;
(*

## 5.2.1.2 composite_sheet_representation

A composite_sheet_representation is a type of shape_representation that implements the ARM concept of Composite_sheet_representation.

EXPRESS specification:

*)
ENTITY composite_sheet_representation
  SUBTYPE OF (shape_representation);
WHERE
  WR1: SIZEOF(['AIC_GEOMETRICALLY_BOUNDED_SURFACE.GEOMETRICALLY_BOUNDED_SURFACE_SHAPE_REPRESENTATION', 'AIC_MANIFOLD_SURFACE.MANIFOLD_SURFACE_SHAPE_REPRESENTATION'] * TYPEOF(SELF)) = 1;
END_ENTITY;
(*

Formal propositions:

WR1: Any instance of composite_sheet_representation shall be either of type geometrically_bounded_surface_shape_representation, or of typemanifold_surface_shape_representation.

## 5.2.1.3 flat_pattern_ply_representation_relationship

A flat_pattern_ply_representation_relationship is a type of shape_representation_relationship.

EXPRESS specification:

*)
ENTITY flat_pattern_ply_representation_relationship
  SUBTYPE OF (shape_representation_relationship);
WHERE
  WR1: 'PRODUCT_PROPERTY_REPRESENTATION_SCHEMA.SHAPE_REPRESENTATION' IN (TYPEOF(SELF\representation_relationship.rep_1) * TYPEOF(SELF\representation_relationship.rep_2));
  WR2: SELF\representation_relationship.rep_1.context_of_items\geometric_representation_context.coordinate_space_dimension = 3;
END_ENTITY;
(*

Formal propositions:

WR1: The rep_1 and rep_2 shall be shape_representations.

WR2: The rep_1 shall always be 3D.

*)
END_SCHEMA;  -- Composite_constituent_shape_mim
(*

© ISO 2019 — All rights reserved