## 4.1 Required AM ARMs

The following EXPRESS interface statements specify the elements imported from the ARMs of other application modules.

EXPRESS specification:

*)
USE FROM Layered_interconnect_complex_template_arm;    --  ISO/TS 10303-1716

USE FROM Package_arm;    --  ISO/TS 10303-1707

REFERENCE FROM Support_resource_arm    --  ISO/TS 10303-1800
  (bag_to_set); 
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Layered_interconnect_complex_template_arm | ISO/TS 10303-1716 |
| --- | --- |
| Package_arm | ISO/TS 10303-1707 |
| Support_resource_arm | ISO/TS 10303-1800 |

NOTE 2   See Annex C, Figures C.1and C.2 for a graphical representation of this schema.

## 4.2 ARM entity definitions

This subclause specifies the ARM entities for this module. Each ARM application entity is an atomic element that embodies a unique application concept and contains attributes specifying the data elements of the entity. The ARM entities and definitions are specified below.

## 4.2.1 Breakout_footprint_definition

A Breakout_footprint_definition is a type of Generic_footprint_definition provided to support re-use of patterns necessary to adapt a footprint to a design layout. A Breakout_footprint_definition specifies the footprint being adapted as well as the adaptation pattern.

EXPRESS specification:

*)
ENTITY Breakout_footprint_definition
  SUBTYPE OF (Generic_footprint_definition);
  reference_footprint : Footprint_definition;
INVERSE
  breakout_traces : SET[1:?] OF stratum_feature_template_location_in_breakout_footprint_definition FOR assembly;
WHERE
  WR1: SELF\Multi_stratum_structured_template.location = reference_footprint\Multi_stratum_structured_template.location;
END_ENTITY;
(*

Attribute definitions:

reference_footprint: specifies the role of the Footprint_definition for the Breakout_footprint_definition. The reference_footprint is the footprint being adapted to a design layout by the Breakout_footprint_definition.

breakout_traces: specifies an inverse relationship that specifies that the existence of the Breakout_footprint_definition is dependent on the existence of theStratum_feature_template_location_in_breakout_footprint_definition that specifies the Breakout_footprint_definition as its assembly. There shall be one or more Stratum_feature_template_location_in_breakout_footprint_definition for a Breakout_footprint_definition. The breakout_traces are the adaptation pattern components. Each component overlaps with a Land instance in the reference_footprint when the Breakout_footprint_definition is instantiated and the geometry rendered in a design layout.

Formal propositions:

WR1: The location of Breakout_footprint_definition shall be same as location of its reference_footprint.

## 4.2.2 Breakout_footprint_definition_shape_model

A Breakout_footprint_definition_shape_model is a type of Structured_template_planar_shape_model.

EXPRESS specification:

*)
ENTITY Breakout_footprint_definition_shape_model
  SUBTYPE OF (Structured_template_planar_shape_model);
  SELF\structured_template_planar_shape_model.shape_characterized_definition : SET[1:1] OF Breakout_footprint_definition;
  reference_shape : Footprint_definition_shape_model;
WHERE
  WR1: reference_shape.shape_characterized_definition[1] :=: shape_characterized_definition[1].reference_footprint;
END_ENTITY;
(*

Attribute definitions:

shape_characterized_definition: specifies the role of the Breakout_footprint_definition for the Breakout_footprint_definition_shape_model. There shall exist exactly one Breakout_footprint_definition for the Breakout_footprint_definition_shape_model.

reference_shape: specifies the role of the Footprint_definition_shape_model for the Breakout_footprint_definition_shape_model.

Formal propositions:

WR1: The footprint referenced by shape_characterized_definition of reference_shape shall be same footprint as that referenced by reference_footprint of shape_characterized_definition of Breakout_footprint_definition_shape_model.

## 4.2.3 Footprint_definition

A Footprint_definition is a type of Generic_footprint_definition. A Footprint_definition is an explicit geometric pattern that when realized in a manufactured interconnect substrate provides an interface to a discrete component when the substrate and the component are combined in an assembly. The specific properties required in a manufacturing process for the end product help determine properties of the pattern.

NOTE    A Footprint_definition uses generic categories when referencing layers and does not contain design specific layer sequences.

EXAMPLE 1   Figure 1 illustrates two categories of integrated circuit footprints. The parameters shown in the illustration could be conveyed using geometric annotation data supported by ISO 10303-1051, but that annotation is optional since this part of ISO 10303 supports explicit geometry data.

Figure 1 —  Typical integrated circuit footprint definition

EXAMPLE 2   Figure 2 illustrates two categories of simple footprints.

Figure 2 —  Typical chip component footprint definition

EXPRESS specification:

*)
ENTITY Footprint_definition
  SUBTYPE OF (Generic_footprint_definition);
  reference_package : OPTIONAL Part_usage_view;
DERIVE
  locations_with_reference_feature : SET OF Part_feature_based_template_location := QUERY(trf <* templates | 'FOOTPRINT_DEFINITION_ARM.PART_FEATURE_BASED_TEMPLATE_LOCATION' IN TYPEOF(trf));
WHERE
  WR1: SIZEOF(QUERY(shape <* SELF\Structured_template.shapes | NOT ('LAYERED_INTERCONNECT_COMPLEX_TEMPLATE_ARM.FOOTPRINT_DEFINITION_SHAPE_MODEL' IN TYPEOF(shape)))) = 0;
  WR2: SIZEOF(QUERY(tlist <* SELF\Structured_template.templates | ('FOOTPRINT_DEFINITION_ARM.FOOTPRINT_DEFINITION' IN TYPEOF(tlist.template)))) = 0;
  WR3: (SIZEOF(locations_with_reference_feature) = 0) OR NOT EXISTS(reference_package) OR (SIZEOF(QUERY(lwrf <* locations_with_reference_feature | NOT (reference_package :=: lwrf\Part_feature_based_template_location.reference_feature\Part_feature.associated_definition))) = 0);
END_ENTITY;
(*

Attribute definitions:

reference_package: specifies the role of the Part_usage_view for the Footprint_definition. The reference_package supports the case where traceability is desired to the originating interface requirement. The existence of this attribute does not limit the application of a Footprint_definition to support other components. A Part_usage_view is specified to support the general case, rather than the typical case of Package. The value of this attribute need not be specified.

locations_with_reference_feature: specifies the set of Part_feature_based_template_locations for the Footprint_definition.

Formal propositions:

WR1: All shapes of Footprint_definition shall be of type Footprint_definition_shape_model.

WR2: The Footprint_definition shall not be composed of other Footprint_definitions.

WR3: If the reference package and the reference features are provided, the reference package shall be referenced by the reference features.

## 4.2.4 Footprint_definition_shape_model

A Footprint_definition_shape_model is a type of Structured_template_planar_shape_model. A Footprint_definition_shape_model is the planar shape of aFootprint_definition. A Footprint_definition_shape_model may reference either a shape of a Part_view_definition or a shape of a Printed_part_template as a causal shape, with which the Footprint_definition_shape_model is to be compatible.

EXPRESS specification:

*)
ENTITY Footprint_definition_shape_model
  SUBTYPE OF (Structured_template_planar_shape_model);
  SELF\structured_template_planar_shape_model.shape_characterized_definition : SET[1:1] OF Footprint_definition;
  reference_shape : OPTIONAL physical_unit_shape_model_select;
  reference_seating_plane_placement : OPTIONAL Usage_concept_usage_relationship;
WHERE
  WR1: NOT (SIZEOF(['PHYSICAL_UNIT_3D_SHAPE_ARM.PHYSICAL_UNIT_3D_SHAPE_MODEL', 'PHYSICAL_UNIT_2D_SHAPE_ARM.PHYSICAL_UNIT_PLANAR_SHAPE_MODEL'] * TYPEOF(reference_shape)) = 1) OR NOT ('PHYSICAL_UNIT_USAGE_VIEW_ARM.PART_USAGE_VIEW' IN TYPEOF(reference_shape\Physical_unit_shape_model.shape_characterized_definition)) OR NOT ('LAYERED_INTERCONNECT_COMPLEX_TEMPLATE_ARM.FOOTPRINT_DEFINITION' IN TYPEOF(shape_characterized_definition[1])) OR (shape_characterized_definition[1]\Footprint_definition.reference_package :=: reference_shape\Physical_unit_shape_model.shape_characterized_definition);
  WR2: NOT (SIZEOF(['PHYSICAL_UNIT_3D_SHAPE_ARM.PHYSICAL_UNIT_3D_SHAPE_MODEL', 'PHYSICAL_UNIT_2D_SHAPE_ARM.PHYSICAL_UNIT_PLANAR_SHAPE_MODEL'] * TYPEOF(reference_shape)) = 1) OR ('PHYSICAL_UNIT_USAGE_VIEW_ARM.PART_USAGE_VIEW' IN TYPEOF(reference_shape\Physical_unit_shape_model.shape_characterized_definition));
  WR3: NOT EXISTS(SELF\Representation.description);
  WR4: NOT EXISTS(reference_shape) OR EXISTS(shape_characterized_definition[1]\Footprint_definition.reference_package);
  WR5: NOT EXISTS(reference_seating_plane_placement) OR EXISTS(reference_shape);
  WR6: NOT EXISTS(reference_seating_plane_placement) OR ('PACKAGE_ARM.PACKAGE' IN TYPEOF(shape_characterized_definition[1]\Footprint_definition.reference_package));
  WR7: NOT EXISTS(reference_seating_plane_placement) OR ('NON_FEATURE_SHAPE_ELEMENT_ARM.SEATING_PLANE' IN TYPEOF(reference_seating_plane_placement\Usage_concept_usage_relationship.associated_usage));
  WR8: NOT EXISTS(reference_seating_plane_placement) OR ((reference_seating_plane_placement\Usage_concept_usage_relationship.associating_usage_shape\Usage_view_level_non_feature_shape_model.model_shape) = reference_shape);
  WR9: NOT EXISTS(reference_seating_plane_placement) OR ((reference_seating_plane_placement\Usage_concept_usage_relationship.associated_usage\Non_feature_shape_element.associated_definition) = shape_characterized_definition[1]\Footprint_definition.reference_package);
  WR10: NOT EXISTS(reference_seating_plane_placement) OR ('PHYSICAL_UNIT_3D_SHAPE_ARM.PHYSICAL_UNIT_3D_SHAPE_MODEL' IN TYPEOF(reference_seating_plane_placement\Usage_concept_usage_relationship.associating_usage_shape\Usage_view_level_non_feature_shape_model.model_shape));
  WR11: NOT (EXISTS(reference_seating_plane_placement) AND ('PHYSICAL_UNIT_2D_SHAPE_ARM.PHYSICAL_UNIT_PLANAR_SHAPE_MODEL' IN TYPEOF(reference_shape)));
  WR12: NOT ('PHYSICAL_UNIT_2D_SHAPE_ARM.PHYSICAL_UNIT_PLANAR_SHAPE_MODEL' IN TYPEOF(reference_shape)) OR (SELF\Geometric_model.context_of_items = reference_shape\Geometric_model.context_of_items);
END_ENTITY;
(*

Attribute definitions:

shape_characterized_definition: specifies the role of the Footprint_definition for the Footprint_definition_shape_model.

reference_shape: specifies the physical_unit_shape_model_select for the Footprint_definition_shape_model. The value of this attribute need not be specified.

reference_seating_plane_placement: specifies the role of the Usage_concept_usage_relationship for the Footprint_definition_shape_model. The value of this attribute need not be specified.

Formal propositions:

WR1: If the reference_shape is a Physical_unit_3d_shape_model or is a Physical_unit_planar_shape_model and if the shape_characterized_definitionattribute of reference_shape references a Part_usage_view and if shape_characterized_definition is a Footprint_definition, then that Part_usage_view shall also be referenced by the reference_package attribute of the shape_characterized_definition.

WR2: If the reference_shape is a Physical_unit_3d_shape_model or a Physical_unit_planar_shape_model, then only a Part_usage_view shall be referenced by the reference_shape.

WR3: The description shall not be populated.

WR4: If reference_shape is provided then the reference_package shall be provided for the associated Footprint_definition.

WR5: If reference_seating_plane_placement is provided then reference_shape shall be provided.

WR6: If reference_seating_plane_placement is provided then the reference_package for the associated Footprint_definition shall be a Package.

WR7: If reference_seating_plane_placement is provided then the associated_usage for the placement shall be a Seating_plane.

WR8: If reference_seating_plane_placement is provided then the geometric model the Seating_plane shape is placed in shall specify the reference_shape as its model_shape.

WR9: If reference_seating_plane_placement is provided then the scope of the geometric model the Seating_plane shape is placed in shall be thereference_package of the associated Footprint_definition.

WR10: If reference_seating_plane_placement is provided then the relevant model shape shall be a three-dimensional model.

WR11: The reference_seating_plane_placement shall not be provided for a member of Footprint_definition_shape_model when the reference_shape is a two-dimensional model. The reference_shape shall not be a two-dimensional model when the reference_seating_plane_placement is provided.

WR12: If the reference_shape is a two-dimensional model then the context of that model shall be the context of this model.

## 4.2.5 Generic_footprint_definition

A Generic_footprint_definition is a type of Multi_stratum_structured_template. A Generic_footprint_definition is either a Footprint_definition or aBreakout_footprint_definition.

EXPRESS specification:

*)
ENTITY Generic_footprint_definition ABSTRACT
  SUPERTYPE OF (ONEOF (Footprint_definition,
                       Breakout_footprint_definition))
  SUBTYPE OF (Multi_stratum_structured_template);
DERIVE
  padstack_based_location : template_arrangement := get_template_arrangement(padstacks);
  padstacks : SET[1:?] OF Template_location_in_structured_template := QUERY(tlict <* SELF\Structured_template.templates | ('LAYERED_INTERCONNECT_COMPLEX_TEMPLATE_ARM.PADSTACK_DEFINITION' IN TYPEOF(tlict.template)));
  single_stratum_components : SET[0:?] OF Single_stratum_template := fd_get_single_stratum_components(SELF\Structured_template.templates);
WHERE
  WR1: SIZEOF(QUERY(ps <* SELF\Structured_template.templates | (ps\Template_location_in_structured_template.template\Multi_stratum_structured_template.location <> SELF\Multi_stratum_structured_template.location) AND (ps\Template_location_in_structured_template.template\Multi_stratum_structured_template.location <> template_arrangement.symmetrical))) = 0;
  WR2: ((SELF\Multi_stratum_structured_template.location = template_arrangement.top) AND (SELF\Product_view_definition.initial_context\View_definition_context.description = 'top')) OR (NOT (SELF\Multi_stratum_structured_template.location = template_arrangement.top) AND NOT (SELF\Product_view_definition.initial_context\View_definition_context.description = 'top'));
  WR3: ((SELF\Multi_stratum_structured_template.location = template_arrangement.bottom) AND (SELF\Product_view_definition.initial_context\View_definition_context.description = 'bottom')) OR (NOT (SELF\Multi_stratum_structured_template.location = template_arrangement.bottom) AND NOT (SELF\Product_view_definition.initial_context\View_definition_context.description = 'bottom'));
  WR4: ((SELF\Multi_stratum_structured_template.location = template_arrangement.symmetrical) AND (SELF\Product_view_definition.initial_context\View_definition_context.description = 'symmetrical')) OR (NOT (SELF\Multi_stratum_structured_template.location = template_arrangement.symmetrical) AND NOT (SELF\Product_view_definition.initial_context\View_definition_context.description = 'symmetrical'));
  WR5: ((SELF\Multi_stratum_structured_template.location = template_arrangement.swappable) AND (SELF\Product_view_definition.initial_context\View_definition_context.description = 'swappable')) OR (NOT (SELF\Multi_stratum_structured_template.location = template_arrangement.swappable) AND NOT (SELF\Product_view_definition.initial_context\View_definition_context.description = 'swappable'));
  WR6: NOT (SIZEOF(single_stratum_components) = 0) OR (SELF\Multi_stratum_structured_template.location = padstack_based_location);
END_ENTITY;
(*

Attribute definitions:

padstack_based_location: specifies the template_arrangement that is evaluated using function get_template_arrangement with padstacks as an argument.

padstacks: specifies the role of Template_location_in_structured_template for the Generic_footprint_definition. There shall be at least oneTemplate_location_in_structured_template provided.

single_stratum_components: specifies the role of the Single_stratum_template for the Generic_footprint_definition. The Single_stratum_templates are optional and need not be provided for the Generic_footprint_definition.

Formal propositions:

WR1: The location of Multi_stratum_structured_templates composing Generic_footprint_definition shall be either symmetrical or same as one of Generic_footprint_definition.

WR2: The location shall be top if and only if the padstack location is top.

WR3: The location shall be bottom if and only if the padstack location is bottom.

WR4: The location shall be symmetrical if and only if the padstack location is symmetrical.

WR5: The location shall be swappable if and only if the padstack location is swappable.

WR6: If there are no single_stratum_components, then the location inherited from the Multi_stratum_structured_template shall be equal to the calculatedpadstack_based_location.

## 4.2.6 Package_footprint_relationship_definition

A Package_footprint_relationship_definition is a type of Product_view_definition provided to explicitly define the geometric relationship between a planar shape of a Footprint_definition and a planar shape of a Package. The Package_footprint_relationship_definition allows an application to archive and exchange many to many relationships between Package and Footprint_definition models.

EXPRESS specification:

*)
ENTITY Package_footprint_relationship_definition
  SUBTYPE OF (Product_view_definition);
DERIVE
  associated_model : SET[1:1] OF Geometric_model := fd_get_amodel(SELF);
  related_models : SET[2:2] OF Geometric_model := fd_get_rmodels(related_elements);
  related_transforms : SET[2:2] OF Geometric_model_relationship_with_transformation := fd_get_transforms(associated_model, related_models);
INVERSE
  related_elements : SET[2:2] OF view_definition_relationship FOR related_view;
WHERE
  WR1: SIZEOF(QUERY(re <* related_elements | ('PACKAGE_ARM.PACKAGE' IN TYPEOF(re\View_definition_relationship.relating_view)))) = 1;
  WR2: SIZEOF(QUERY(re <* related_elements | ('FOOTPRINT_DEFINITION_ARM.FOOTPRINT_DEFINITION' IN TYPEOF(re\View_definition_relationship.relating_view)))) = 1;
  WR3: SIZEOF(related_models) = 2;
  WR4: SIZEOF(associated_model) = 1;
  WR5: SIZEOF(QUERY(rm <* related_models | ('PHYSICAL_UNIT_2D_SHAPE_ARM.PHYSICAL_UNIT_PLANAR_SHAPE_MODEL' IN TYPEOF(rm)))) = 1;
  WR6: SIZEOF(QUERY(rm <* related_models | ('FOOTPRINT_DEFINITION_ARM.FOOTPRINT_DEFINITION_SHAPE_MODEL' IN TYPEOF(rm)))) = 1;
  WR7: 'LAYERED_2D_SHAPE_ARM.PLANAR_SHAPE_MODEL' IN TYPEOF(associated_model[1]);
  WR8: SIZEOF(related_transforms) = 2;
END_ENTITY;
(*

Attribute definitions:

associated_model: specifies a Geometric_model that is establishing a common context in which the Package model and the Footprint_definition model are related. Each of the two models is independently transformed into the associated_model, with the role of the transforms being to establish the formal relationship.

related_models: specifies two Geometric_models, one of which is a planar model for a Package, and the other is a planar model for a Footprint_definition. The models are individually associated to the Package and Footprint_definition related to the Package_footprint_relationship_definition by related_elementsthrough a Shape_description_association.

related_transforms: specifies two Geometric_model_relationship_with_transformation that individually specify the transformation matrix that locates aPackage model or a Footprint_definition model in the associated_model.

NOTE 1   Taken together, the related_transforms specifies the geometric relationship between geometric elements in a Package model to geometric elements in a Footprint_definition model.

related_elements: is an inverse attribute that specifies that the Package_footprint_relationship_definition is existence dependent on two members ofView_definition_relationship. One member of View_definition_relationship references a Package. The other member references a Footprint_definition.

NOTE 2   The direction of the related and relating attributes differs from an assembly relationship since a Package_footprint_relationship_definition is not an assembly.

Formal propositions:

WR1: At least one member of Package shall be included in related_elements.

WR2: At least one member of Footprint_definition shall be included in related_elements.

NOTE 3   WR1 with WR2 and the population constraint on related_elements ensure that there is one Package and one Footprint_definition related to a member ofPackage_footprint_relationship_definition.

WR3: There shall be exactly two models related to the Package_footprint_relationship_definition model.

WR4: There shall be exactly one associated_model.

WR5: One of the related_models shall be a Physical_unit_planar_shape_model.

WR6: One of the related_models shall be a Footprint_definition_shape_model.

NOTE 4   WR3, WR5, WR6 together ensure that there are the correct type of models in related_models.

WR7: The associated_model shall have a dimensionality of 2.

WR8: There shall exist exactly two related_transforms.

## 4.2.7 Part_feature_based_template_location

A Part_feature_based_template_location is a type of Template_location_in_structured_template that supports the case that properties of a specific element of a Footprint_definition are dependent on properties of a specific Part_feature.

EXPRESS specification:

*)
ENTITY Part_feature_based_template_location
  SUBTYPE OF (Template_location_in_structured_template);
  reference_feature : Part_feature;
END_ENTITY;
(*

Attribute definitions:

reference_feature: specifies the role of one Part_feature for the Part_feature_based_template_location.

## 4.2.8 Stratum_feature_template_location_in_breakout_footprint_definition

A Stratum_feature_template_location_in_breakout_footprint_definition is a type of Stratum_specific_template_location that also provides an explicit map between a padstack location in a Breakout_footprint_definition and the source requirement padstack location in a Footprint_definition.

EXPRESS specification:

*)
ENTITY Stratum_feature_template_location_in_breakout_footprint_definition
  SUBTYPE OF (Stratum_specific_template_location);
  SELF\template_location_in_structured_template.assembly : Breakout_footprint_definition;
  SELF\template_location_in_structured_template.template : Stratum_feature_template;
  original_padstack : Part_feature_based_template_location;
  breakout_padstack : Template_location_in_structured_template;
WHERE
  WR1: original_padstack.assembly :=: assembly.reference_footprint;
  WR2: breakout_padstack.assembly :=: assembly;
END_ENTITY;
(*

Attribute definitions:

assembly: specifies the role of the Breakout_footprint_definition for the Stratum_feature_template_location_in_breakout_footprint_definition.

template: specifies the role of the Stratum_feature_template for the Stratum_feature_template_location_in_breakout_footprint_definition.

original_padstack: specifies the role of the Part_feature_based_template_location for the Stratum_feature_template_location_in_breakout_footprint_definition.

breakout_padstack: specifies the role of the Template_location_in_structured_template for the Stratum_feature_template_location_in_breakout_footprint_definition.

Formal propositions:

WR1: The original_padstack.assembly shall be the same member as assembly.reference_footprint.

WR2: The breakout_padstack.assembly shall be the same member as assembly.

## 4.3 ARM subtype constraint definitions

This subclause specifies the ARM subtype constraints for this module. Each subtype constraint places constraints on the possible super-type / subtype instantiations. The ARM subtype constraints and definitions are specified below.

## 4.3.1 structured_template_planar_shape_model_subtypes

The structured_template_planar_shape_model_subtypes constraint specifies a constraint that applies to instances ofstructured_template_planar_shape_model and enforces the rule that its subtypes and Footprint_definition_shape_model are exclusive.

EXPRESS specification:

*)
SUBTYPE_CONSTRAINT structured_template_planar_shape_model_subtypes FOR structured_template_planar_shape_model;
  (ONEOF (Breakout_footprint_definition_shape_model,
          Footprint_definition_shape_model));
END_SUBTYPE_CONSTRAINT;
(*

## 4.3.2 multi_stratum_structured_template_subtypes

The multi_stratum_structured_template_subtypes constraint specifies a constraint that applies to instances of multi_stratum_structured_template and enforces the rule that its subtypes , Generic_footprint_definition and Multi_stratum_special_symbol_template are exclusive.

EXPRESS specification:

*)
SUBTYPE_CONSTRAINT multi_stratum_structured_template_subtypes FOR multi_stratum_structured_template;
  (ONEOF (Padstack_definition,
          Generic_footprint_definition,
          Multi_stratum_special_symbol_template));
END_SUBTYPE_CONSTRAINT;
(*

## 4.4 ARM function definitions

This subclause specifies the ARM functions for this module. The ARM functions and definitions are specified below.

## 4.4.1 fd_get_components

The fd_get_components function returns the set of Product_view_definitions that are referenced by the relating_view attribute of the input.

EXPRESS specification:

*)
FUNCTION fd_get_components (input : SET[2:2] OF View_definition_relationship) : SET OF Product_view_definition;

LOCAL

pvd : SET OF Product_view_definition := [];

END_LOCAL;

REPEAT i := 1 TO HIINDEX(input);

pvd := pvd + input[i]\View_definition_relationship.relating_view;

END_REPEAT;

RETURN(pvd);

END_FUNCTION; 
(*

Argument definitions:

input: the specified set of View_definition_relationships

## 4.4.2 fd_get_amodel

The fd_get_amodel function returns the set of Geometric_model that are linked to a Product_view_definition.

EXPRESS specification:

*)
FUNCTION fd_get_amodel (input : Product_view_definition) : SET OF Geometric_model;

LOCAL

gm : SET OF Geometric_model := [];

sda : SET OF Shape_description_association := [];

END_LOCAL;

-- process input.primary_shape_representation

IF 'ELEMENTAL_GEOMETRIC_SHAPE_ARM.GEOMETRIC_MODEL' IN TYPEOF(input.primary_shape_representation)

THEN

gm := gm + input.primary_shape_representation;

END_IF;

-- process input.auxiliary_shape_representations

IF (SIZEOF(input.auxiliary_shape_representations) >= 0)

THEN

REPEAT i := 1 TO HIINDEX(input.auxiliary_shape_representations);

IF 'ELEMENTAL_GEOMETRIC_SHAPE_ARM.GEOMETRIC_MODEL' IN TYPEOF(input.auxiliary_shape_representations[i])

THEN

gm := gm + input.auxiliary_shape_representations[i];

END_IF;

END_REPEAT;

END_IF;

-- process other SHAPE_DESCRIPTION_ASSOCIATIONs that could still hang around

sda := bag_to_set(USEDIN(input, 'SHAPE_PROPERTY_ASSIGNMENT_ARM.SHAPE_DESCRIPTION_ASSOCIATION.DEFINITION'));

IF (SIZEOF(sda) >= 0)

THEN

REPEAT i := 1 TO HIINDEX(sda);

IF 'ELEMENTAL_GEOMETRIC_SHAPE_ARM.GEOMETRIC_MODEL' IN TYPEOF(input.primary_shape_representation)

THEN

gm := gm + sda[i]\Property_definition_representation.used_representation;

END_IF;

END_REPEAT;

END_IF;

RETURN(gm);

END_FUNCTION; 
(*

Argument definitions:

input: the specified Product_view_definition.

## 4.4.3 fd_get_rmodels

The fd_get_rmodels function returns the set of Geometric_models that are related to the Product_view_definitions that are related by the input set.

EXPRESS specification:

*)
FUNCTION fd_get_rmodels (input : SET[2:2] OF View_definition_relationship) : SET OF Geometric_model;

LOCAL

gm : SET OF Geometric_model := [];

pvd : SET OF Product_view_definition := [];

END_LOCAL;

pvd := fd_get_components(input);

IF (SIZEOF(pvd) >= 0)

THEN

REPEAT i := 1 TO HIINDEX(pvd);

gm := gm + fd_get_amodel(pvd[i]);

END_REPEAT;

RETURN(gm);

END_IF;

RETURN(?);

END_FUNCTION; 
(*

Argument definitions:

input: the specified set of View_definition_relationship.

## 4.4.4 fd_get_single_stratum_components

The fd_get_single_stratum_components function returns the set of Single_stratum_template that are in the role of template for the input.

EXPRESS specification:

*)
FUNCTION fd_get_single_stratum_components (input : SET OF Template_location_in_structured_template) : SET OF Single_stratum_template;

LOCAL

sst : SET OF Single_stratum_template := [];

END_LOCAL;

REPEAT i := 1 TO HIINDEX(input);

IF ('LAYERED_INTERCONNECT_SIMPLE_TEMPLATE_ARM.SINGLE_STRATUM_TEMPLATE' IN

TYPEOF(input[i]\Template_location_in_structured_template.template))

THEN

sst := sst + input[i]\Template_location_in_structured_template.template;

END_IF;

END_REPEAT;

RETURN(sst);

END_FUNCTION; 
(*

Argument definitions:

input: the specified set of Template_location_in_structured_template.

## 4.4.5 get_template_arrangement

The get_template_arrangement function gets the template_arrangement value that is consistent with all Template_location_in_structured_templatespecified by input.

It returns top if there is at least one Template_location_in_structured_template specified in input that has location ofTemplate_location_in_structured_template set to top and there are no Template_location_in_structured_template specified in input that have location ofTemplate_location_in_structured_template set to bottom.

It returns bottom if there is at least one Template_location_in_structured_template specified in input that has location ofTemplate_location_in_structured_template set to bottom and there are no Template_location_in_structured_template specified in input that have locationof Template_location_in_structured_template set to top.

It returns symmetrical if there is at least one Template_location_in_structured_template specified in input that has location ofTemplate_location_in_structured_template set to symmetrical and there are no Padstack_location_in_footprint_definition specified in input that havelocation of Template_location_in_structured_template set to top or bottom.

It returns Undefined otherwise.

EXPRESS specification:

*)
FUNCTION get_template_arrangement (input : SET OF Template_location_in_structured_template) : template_arrangement;

LOCAL

tlist : SET OF Template_location_in_structured_template := (QUERY(inx

<* input

| ('LAYERED_INTERCONNECT_COMPLEX_TEMPLATE_ARM.MULTI_STRATUM_STRUCTURED_TEMPLATE' IN

TYPEOF(inx.template))));

pabt : BAG OF STRING := [];

pabb : BAG OF STRING := [];

pabsymmetrical : BAG OF STRING := [];

pabswappable : BAG OF STRING := [];

END_LOCAL;

REPEAT i := 1 TO SIZEOF(tlist) BY 1;

IF (tlist[i].template\Multi_stratum_structured_template.location = template_arrangement.top)

THEN

pabt := pabt + 'top';

END_IF;

IF (tlist[i].template\Multi_stratum_structured_template.location = template_arrangement.bottom)

THEN

pabb := pabb + 'bottom';

END_IF;

IF (tlist[i].template\Multi_stratum_structured_template.location = template_arrangement.symmetrical)

THEN

pabsymmetrical := pabsymmetrical + 'symmetrical';

END_IF;

IF (tlist[i].template\Multi_stratum_structured_template.location = template_arrangement.swappable)

THEN

pabswappable := pabswappable + 'swappable';

END_IF;

END_REPEAT;

IF ((SIZEOF(pabt) > 0) AND (SIZEOF(pabb) = 0))

THEN

RETURN(template_arrangement.top);

END_IF;

IF ((SIZEOF(pabb) > 0) AND (SIZEOF(pabt) = 0))

THEN

RETURN(template_arrangement.bottom);

END_IF;

IF ((SIZEOF(pabb) = 0) AND (SIZEOF(pabt) = 0) AND (SIZEOF(pabsymmetrical) > 0))

THEN

RETURN(template_arrangement.symmetrical);

END_IF;

IF ((SIZEOF(pabb) = 0) AND (SIZEOF(pabt) = 0) AND (SIZEOF(pabsymmetrical) = 0) AND (SIZEOF(pabswappable) > 0))

THEN

RETURN(template_arrangement.swappable);

END_IF;

RETURN(?);

END_FUNCTION; 
(*

Argument definitions:

input: the specified set of Padstack_location_in_footprint_definitions.

## 4.4.6 fd_get_transforms

The fd_get_transforms function returns the set of Geometric_model_relationship_with_transformation that transform the elements of input2 to the domain of input1.

EXPRESS specification:

*)
FUNCTION fd_get_transforms (input1 : SET[1:1] OF Geometric_model; input2 : SET[2:2] OF Geometric_model) : SET OF Geometric_model_relationship_with_transformation;

LOCAL

gmrwt : SET OF Geometric_model_relationship_with_transformation := [];

END_LOCAL;

gmrwt := bag_to_set(USEDIN(input1[1], 'GEOMETRIC_MODEL_RELATIONSHIP_ARM.GEOMETRIC_MODEL_RELATIONSHIP.REP_1'));

REPEAT i := 1 TO HIINDEX(gmrwt);

IF NOT ((gmrwt[i]\Geometric_model_relationship.rep_2 IN input2) AND

('CONTEXTUAL_SHAPE_POSITIONING_ARM.GEOMETRIC_MODEL_RELATIONSHIP_WITH_TRANSFORMATION' IN TYPEOF(gmrwt[i])))

THEN

RETURN(?);

END_IF;

END_REPEAT;

RETURN(gmrwt);

END_FUNCTION; 
(*

Argument definitions:

input1: the specified set of Geometric_models.

input2: the specified set of Geometric_models.

*)
END_SCHEMA;  -- Footprint_definition_arm
(*

© ISO 2019 — All rights reserved