## 5.2 MIM EXPRESS short listing

This clause specifies the EXPRESS schema derived from the mapping table. It uses elements from the common resources or from other application modules and defines the EXPRESS constructs that are specific to this part of ISO 10303.

This clause constitutes the Module Interpreted Module (MIM) of the application module.

This clause also specifies the modifications that apply to the constructs imported from the common resources.

The following restrictions apply to the use, in this schema, of constructs defined in common resources or in application modules:

- use of a supertype entity does not make applicable any of its specializations, unless the specialization is also imported in the MIM schema;

- use of a SELECT type does not make applicable any of its listed types unless the listed type is also imported in the MIM schema.

Short names of entities defined in this schema are described in Annex A. Unambiguous identification of this schema is defined in Annex B.

EXPRESS specification: 

*)
SCHEMA Product_as_individual_assembly_and_test_mim;

USE FROM Activity_mim;    --  ISO/TS 10303-1047

USE FROM Planned_characteristic_mim;    --  ISO/TS 10303-1733

USE FROM Process_plan_mim;    --  ISO/TS 10303-1803

USE FROM Product_as_individual_mim;    --  ISO/TS 10303-1164

USE FROM Property_assignment_mim;    --  ISO/TS 10303-1030
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Activity_mim | ISO/TS 10303-1047 |
| --- | --- |
| Planned_characteristic_mim | ISO/TS 10303-1733 |
| Process_plan_mim | ISO/TS 10303-1803 |
| Product_as_individual_mim | ISO/TS 10303-1164 |
| Property_assignment_mim | ISO/TS 10303-1030 |

NOTE 2   See Annex D, Figures D.1and D.2 for a graphical representation of this schema.

## 5.2.1 MIM type definition

This subclause specifies the MIM type for this application module. The MIM type and definition is specified below.

## 5.2.1.1 paiaata_action_item

The paiaata_action_item type is an extension of the action_items type. It adds the data type property_definition to the list of alternate data types.

EXPRESS specification:

*)
TYPE paiaata_action_item = SELECT BASED_ON action_items WITH 
   (property_definition);
END_TYPE; 
(*

## 5.2.2 MIM entity definition

This subclause specifies the MIM entity for this module. The MIM entity and definition is specified below.

## 5.2.2.1 evaluated_characteristic_of_product_as_individual_test_result

An evaluated_characteristic_of_product_as_individual_test_result is a type of evaluated_characteristic that implements the ARM concept ofEvaluated_characteristic_of_product_as_individual_test_result.

EXPRESS specification:

*)
ENTITY evaluated_characteristic_of_product_as_individual_test_result
  SUBTYPE OF (evaluated_characteristic);
END_ENTITY;
(*

*)
END_SCHEMA;  -- Product_as_individual_assembly_and_test_mim
(*

© ISO 2019 — All rights reserved