## 4.1 ARM type definition

This subclause specifies the ARM type for this application module. The ARM type and definition is specified below.

## 4.1.1 description_item

The description_item type is an extensible list of alternate data types. Additional alternate data types are specified in select data types that extend the description_item type.

NOTE   This empty extensible select requires extension in a further module to ensure that entities that refer to it have at least one valid instantiation.

EXPRESS specification:

*)
TYPE description_item = EXTENSIBLE GENERIC_ENTITY SELECT;
END_TYPE; 
(*

## 4.2 ARM entity definitions

This subclause specifies the ARM entities for this module. Each ARM application entity is an atomic element that embodies a unique application concept and contains attributes specifying the data elements of the entity. The ARM entities and definitions are specified below.

## 4.2.1 Description_text

A Description_text is a set of words that describe something.

EXAMPLE    The same thing may have multiple descriptions associated with it. Relating both a short description and a long description to an operational activity or an organization, as required by the US DoD Architecture Framework, are examples of multiple descriptions applied to the same item.

EXPRESS specification:

*)
ENTITY Description_text;
  description : STRING;
END_ENTITY;
(*

Attribute definitions:

description: the text that constitutes the description

## 4.2.2 Description_text_assignment

A Description_text_assignment is the assignment of a description to something.

EXPRESS specification:

*)
ENTITY Description_text_assignment;
  description : Description_text;
  items : SET[1:?] OF description_item;
END_ENTITY;
(*

Attribute definitions:

description: the text that constitutes the description

items: the set of items being described

## 4.2.3 Description_text_assignment_relationship

A Description_text_assignment_relationship is a relationship between two Description_text_assignment objects.

EXPRESS specification:

*)
ENTITY Description_text_assignment_relationship;
  description : OPTIONAL STRING;
  id : OPTIONAL STRING;
  related : Description_text_assignment;
  relating : Description_text_assignment;
  relation_type : STRING;
WHERE
  WR1: relating :<>: related;
END_ENTITY;
(*

Attribute definitions:

description: the text or the set of texts that provide further information about the Description_text_assignment_relationship. The value of this attribute need not be specified.

id: the identifier or set of identifiers for the Description_text_assignment_relationship. The value of this attribute need not be specified.

related: the other object of Description_text_assignment that is part of the relationship.

relating: one object of Description_text_assignment that is part of the relationship.

relation_type: the meaning of the relationship.

Formal propositions:

WR1: the related and the relating must be different.

*)
END_SCHEMA;  -- Description_assignment_arm
(*

© ISO 2019 — All rights reserved