## 4.1 Required AM ARMs

The following EXPRESS interface statements specify the elements imported from the ARMs of other application modules.

EXPRESS specification:

*)
USE FROM Assembly_constraints_arm;    --  ISO/TS 10303-1794

USE FROM Design_product_data_management_arm;    --  ISO/TS 10303-1628

USE FROM Item_definition_structure_arm;    --  ISO/TS 10303-1345

USE FROM Part_shape_arm;    --  ISO/TS 10303-1807

USE FROM Shape_feature_arm;    --  ISO/TS 10303-1764
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Assembly_constraints_arm | ISO/TS 10303-1794 |
| --- | --- |
| Design_product_data_management_arm | ISO/TS 10303-1628 |
| Item_definition_structure_arm | ISO/TS 10303-1345 |
| Part_shape_arm | ISO/TS 10303-1807 |
| Shape_feature_arm | ISO/TS 10303-1764 |

NOTE 2   See Annex C, Figures C.1, C.2, C.3, C.4and C.5 for a graphical representation of this schema.

## 4.2 ARM type definitions

This subclause specifies the ARM types for this application module. The ARM types and definitions are specified below.

## 4.2.1 assembly_constraint_select

The assembly_constraint_select type allows for the designation of the data types Binary_assembly_constraint, and Fixed_constituent_assembly_constraint.

EXPRESS specification:

*)
TYPE assembly_constraint_select = SELECT
   (Binary_assembly_constraint, 
    Fixed_constituent_assembly_constraint);
END_TYPE; 
(*

## 4.2.2 assembly_constraint_type_enumeration

An assembly_constraint_type_enumeration lists predefined constraint_types for an Assembly_shape_constraint.

EXPRESS specification:

*)
TYPE assembly_constraint_type_enumeration = EXTENSIBLE ENUMERATION OF 
   (coaxial_assembly_constraint,
    complex,
    dimensioned_angle_assembly_constraint,
    dimensioned_parallel_assembly_constraint,
    dimensioned_surface_distance_assembly_constraint,
    fixed_constituent_assembly_constraint,
    incidence_assembly_constraint,
    parallel_assembly_constraint,
    perpendicular_assembly_constraint,
    tangent_assembly_constraint);
END_TYPE; 
(*

Enumerated item definitions:

coaxial_assembly_constraint: specifies that features of constituents of an assembly are constrained to be coaxial.

complex: specifies that features of constituents of an assembly are constrained to be complex in a way that is not covered by any other value of this enumeration.

dimensioned_angle_assembly_constraint: specifies that the angle between two features of constituents of an assembly is constrained to a specific value.

dimensioned_parallel_assembly_constraint: specifies that features of constituents of an assembly are constrained to be parallel at a certain distance.

dimensioned_surface_distance_assembly_constraint: specifies that features of constituents of an assembly are constrained to have a minimal distance.

fixed_constituent_assembly_constraint: specifies that features of constituents of an assembly are constrained to be fixed in space.

incidence_assembly_constraint: specifies that features of constituents of an assembly are constrained so that one feature entirely included all the other features.

parallel_assembly_constraint: specifies that features of constituents of an assembly are constrained to be parallel.

perpendicular_assembly_constraint: specifies that features of constituents of an assembly are constrained to be perpendicular.

tangent_assembly_constraint: specifies that features of constituents of an assembly are constrained to be tangential to each other.

## 4.2.3 assembly_definition_or_assembly_shape_constraint

The assembly_definition_or_assembly_shape_constraint type allows for the designation of the data types Assembly_definition, and Assembly_shape_constraint.

EXPRESS specification:

*)
TYPE assembly_definition_or_assembly_shape_constraint = SELECT
   (Assembly_definition, 
    Assembly_shape_constraint);
END_TYPE; 
(*

## 4.2.4 assembly_definition_or_assembly_shape_joint

The assembly_definition_or_assembly_shape_joint type allows for the designation of the data types Assembly_definition, and Assembly_shape_joint.

EXPRESS specification:

*)
TYPE assembly_definition_or_assembly_shape_joint = SELECT
   (Assembly_definition, 
    Assembly_shape_joint);
END_TYPE; 
(*

## 4.2.5 assembly_shape_constraint_or_joint_definition_select

The assembly_shape_constraint_or_joint_definition_select type allows for the designation of the data types Part_shape_element, Part_shape_element_relationship, Shape_feature_definition, Shape_feature_definition_relationship, Shape_feature_definition_element, and Shape_feature_definition_element_relationship.

EXPRESS specification:

*)
TYPE assembly_shape_constraint_or_joint_definition_select = SELECT
   (Part_shape_element, 
    Part_shape_element_relationship, 
    Shape_feature_definition, 
    Shape_feature_definition_relationship, 
    Shape_feature_definition_element, 
    Shape_feature_definition_element_relationship);
END_TYPE; 
(*

## 4.2.6 connected_terminal_select

The connected_terminal_select type allows for the designation of the data types General_part_terminal, and Occurrence_terminal.

EXPRESS specification:

*)
TYPE connected_terminal_select = SELECT
   (General_part_terminal, 
    Occurrence_terminal);
END_TYPE; 
(*

## 4.2.7 general_assembly_joint_type_enumeration

A general_assembly_joint_type_enumeration lists predefined joint_types for an Assembly_shape_joint.

EXPRESS specification:

*)
TYPE general_assembly_joint_type_enumeration = EXTENSIBLE ENUMERATION ;
END_TYPE; 
(*

## 4.2.8 gdc_part_shape_element_definition_select

The gdc_part_shape_element_definition_select type is an extension of the part_shape_element_definition_select type. It adds the data type Shape_feature_definition_element to the list of alternate data types.

EXPRESS specification:

*)
TYPE gdc_part_shape_element_definition_select = SELECT BASED_ON part_shape_element_definition_select WITH 
   (Shape_feature_definition_element);
END_TYPE; 
(*

## 4.2.9 general_part_terminal_associated_definition_select

The general_part_terminal_associated_definition_select type allows for the designation of the data types Product_specification, General_part_terminal, and Part_view_definition.

EXPRESS specification:

*)
TYPE general_part_terminal_associated_definition_select = SELECT
   (Product_specification, 
    General_part_terminal, 
    Part_view_definition);
END_TYPE; 
(*

## 4.2.10 general_terminal_joint_type_enumeration

A general_terminal_joint_type_enumeration lists predefined intended_joint_types for a General_part_terminal.

EXPRESS specification:

*)
TYPE general_terminal_joint_type_enumeration = EXTENSIBLE ENUMERATION ;
END_TYPE; 
(*

## 4.2.11 join_or_interface_terminal_enumeration

A join_or_interface_terminal_enumeration lists predefined join_or_interface_terminals for a General_part_terminal.

EXPRESS specification:

*)
TYPE join_or_interface_terminal_enumeration = ENUMERATION OF 
   (interface_terminal,
    join_terminal);
END_TYPE; 
(*

Enumerated item definitions:

interface_terminal: specifies the connection type is to interface to a next-higher-assembly level.

join_terminal: specifies the connection type is to join at the same immediate-assembly level.

## 4.2.12 occurrence_contact_feature_associated_definition_select

The occurrence_contact_feature_associated_definition_select type allows for the designation of the data types Occurrence_contact_feature, and Product_occurrence.

EXPRESS specification:

*)
TYPE occurrence_contact_feature_associated_definition_select = SELECT
   (Occurrence_contact_feature, 
    Product_occurrence);
END_TYPE; 
(*

## 4.2.13 occurrence_contact_feature_definition_select

The occurrence_contact_feature_definition_select type allows for the designation of the data types Occurrence_contact_feature, Part_contact_feature, and Contact_feature_definition.

EXPRESS specification:

*)
TYPE occurrence_contact_feature_definition_select = SELECT
   (Occurrence_contact_feature, 
    Part_contact_feature, 
    Contact_feature_definition);
END_TYPE; 
(*

## 4.2.14 occurrence_shape_element_associated_definition_select

The occurrence_shape_element_associated_definition_select type allows for the designation of the data types Occurrence_shape_element, and Product_occurrence.

EXPRESS specification:

*)
TYPE occurrence_shape_element_associated_definition_select = SELECT
   (Occurrence_shape_element, 
    Product_occurrence);
END_TYPE; 
(*

## 4.2.15 occurrence_shape_element_definition_select

The occurrence_shape_element_definition_select type allows for the designation of the data types Occurrence_shape_element, Part_shape_element, and Shape_feature_definition.

EXPRESS specification:

*)
TYPE occurrence_shape_element_definition_select = SELECT
   (Occurrence_shape_element, 
    Part_shape_element, 
    Shape_feature_definition);
END_TYPE; 
(*

## 4.2.16 occurrence_shape_feature_associated_definition_select

The occurrence_shape_feature_associated_definition_select type allows for the designation of the data types Occurrence_shape_feature, and Product_occurrence.

EXPRESS specification:

*)
TYPE occurrence_shape_feature_associated_definition_select = SELECT
   (Occurrence_shape_feature, 
    Product_occurrence);
END_TYPE; 
(*

## 4.2.17 occurrence_shape_feature_definition_select

The occurrence_shape_feature_definition_select type allows for the designation of the data types General_part_feature, Occurrence_shape_feature, and Shape_feature_definition.

EXPRESS specification:

*)
TYPE occurrence_shape_feature_definition_select = SELECT
   (General_part_feature, 
    Occurrence_shape_feature, 
    Shape_feature_definition);
END_TYPE; 
(*

## 4.2.18 occurrence_terminal_associated_definition_select

The occurrence_terminal_associated_definition_select type allows for the designation of the data types Occurrence_terminal, and Product_occurrence.

EXPRESS specification:

*)
TYPE occurrence_terminal_associated_definition_select = SELECT
   (Occurrence_terminal, 
    Product_occurrence);
END_TYPE; 
(*

## 4.2.19 occurrence_terminal_definition_select

The occurrence_terminal_definition_select type allows for the designation of the data types Contact_feature_definition, General_part_terminal, and Occurrence_terminal.

EXPRESS specification:

*)
TYPE occurrence_terminal_definition_select = SELECT
   (Contact_feature_definition, 
    General_part_terminal, 
    Occurrence_terminal);
END_TYPE; 
(*

## 4.2.20 occurrence_transport_feature_associated_definition_select

The occurrence_transport_feature_associated_definition_select type allows for the designation of the data types Occurrence_transport_feature, and Product_occurrence.

EXPRESS specification:

*)
TYPE occurrence_transport_feature_associated_definition_select = SELECT
   (Occurrence_transport_feature, 
    Product_occurrence);
END_TYPE; 
(*

## 4.2.21 occurrence_transport_feature_definition_select

The occurrence_transport_feature_definition_select type allows for the designation of the data types Occurrence_transport_feature, and Part_transport_feature.

EXPRESS specification:

*)
TYPE occurrence_transport_feature_definition_select = SELECT
   (Occurrence_transport_feature, 
    Part_transport_feature);
END_TYPE; 
(*

## 4.2.22 part_contact_feature_associated_definition_select

The part_contact_feature_associated_definition_select type allows for the designation of the data types Part_view_definition, Part_contact_feature, and Product_specification.

EXPRESS specification:

*)
TYPE part_contact_feature_associated_definition_select = SELECT
   (Part_view_definition, 
    Part_contact_feature, 
    Product_specification);
END_TYPE; 
(*

## 4.2.23 part_contact_feature_definition_select

The part_contact_feature_definition_select type allows for the designation of the data types Contact_feature_definition, and Shape_feature_definition_element.

EXPRESS specification:

*)
TYPE part_contact_feature_definition_select = SELECT
   (Contact_feature_definition, 
    Shape_feature_definition_element);
END_TYPE; 
(*

## 4.2.24 part_contact_feature_fit_relationship_definition_select

The part_contact_feature_fit_relationship_definition_select type allows for the designation of the data types Contact_feature_definition_fit_relationship, and Shape_feature_definition_element_relationship.

EXPRESS specification:

*)
TYPE part_contact_feature_fit_relationship_definition_select = SELECT
   (Contact_feature_definition_fit_relationship, 
    Shape_feature_definition_element_relationship);
END_TYPE; 
(*

## 4.2.25 part_feature_fit_relationship_definition_select

The part_feature_fit_relationship_definition_select type allows for the designation of the data types Shape_feature_definition_fit_relationship, and Shape_feature_definition_element_relationship.

EXPRESS specification:

*)
TYPE part_feature_fit_relationship_definition_select = SELECT
   (Shape_feature_definition_fit_relationship, 
    Shape_feature_definition_element_relationship);
END_TYPE; 
(*

## 4.2.26 part_transport_feature_associated_definition_select

The part_transport_feature_associated_definition_select type allows for the designation of the data types Product_specification, Part_transport_feature, and Part_view_definition.

EXPRESS specification:

*)
TYPE part_transport_feature_associated_definition_select = SELECT
   (Product_specification, 
    Part_transport_feature, 
    Part_view_definition);
END_TYPE; 
(*

## 4.2.27 part_view_definition_or_part_shape_element_or_product_specification_select

The part_view_definition_or_part_shape_element_or_product_specification_select type is an extension of the part_view_definition_or_part_shape_element_select type. It adds the data type Product_specification to the list of alternate data types.

EXPRESS specification:

*)
TYPE part_view_definition_or_part_shape_element_or_product_specification_select = SELECT BASED_ONpart_view_definition_or_part_shape_element_select WITH 
   (Product_specification);
END_TYPE; 
(*

## 4.2.28 terminal_and_transport_domain_type

A terminal_and_transport_domain_type lists the domain type values for a General_part_terminal, Part_transport_feature or an Occurrence_transport_feature.

EXPRESS specification:

*)
TYPE terminal_and_transport_domain_type = EXTENSIBLE ENUMERATION OF 
   (electrical,
    thermal,
    optical,
    magnetic,
    matter);
END_TYPE; 
(*

Enumerated item definitions:

electrical: specifies the transport type is electrical;

thermal: specifies the transport type is thermal;

optical: specifies the transport type is optical;

magnetic: specifies the transport type is magnetic.

matter: specifies the transport type is matter such as liquid or gas.

## 4.3 ARM entity definitions

This subclause specifies the ARM entities for this module. Each ARM application entity is an atomic element that embodies a unique application concept and contains attributes specifying the data elements of the entity. The ARM entities and definitions are specified below.

## 4.3.1 Assembly_shape_constraint

An Assembly_shape_constraint is a type of Part_shape_element that defines a constraint between two or more Occurrence_shape_features within an Assembly_definition. An Assembly_shape_constraint may be composed of other Assembly_shape_constraints.

EXPRESS specification:

*)
ENTITY Assembly_shape_constraint
  SUBTYPE OF (Part_shape_element);
  SELF\shape_element.associated_definition : assembly_definition_or_assembly_shape_constraint;
  SELF\part_shape_element.definition : OPTIONAL assembly_shape_constraint_or_joint_definition_select;
  SELF\shape_element.identified_item : OPTIONAL assembly_constraint_select;
  constraint_type : OPTIONAL assembly_constraint_type_enumeration;
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Assembly_definition or of type Assembly_shape_constraint.

definition: a Contact_feature_definition_fit_relationship or a Shape_feature_definition that is the definition for the Assembly_shape_constraint.

identified_item: an inherited attribute that shall have a value of type Binary_assembly_constraint that specifies the geometric details of the constraint. The value of this attribute need not be specified.

constraint_type: characterize the kind of constraint. The value of this attribute need not be specified.

## 4.3.2 Assembly_shape_constraint_item_relationship

An Assembly_shape_constraint_item_relationship is a type of Shape_element_relationship that relates an Occurrence_shape_feature into an Assembly_shape_constraint.

EXPRESS specification:

*)
ENTITY Assembly_shape_constraint_item_relationship
  SUBTYPE OF (Shape_element_relationship);
  SELF\shape_element_relationship.relating : Assembly_shape_constraint;
  SELF\shape_element_relationship.related : Occurrence_shape_feature;
WHERE
  WR1: main_associated_definition(relating\Assembly_shape_constraint.associated_definition) IN main_upper_usage(main_associated_definition(related))\Product_occurrence.occurrence_contexts;
END_ENTITY;
(*

Attribute definitions:

relating: an inherited attribute that shall be of type Assembly_shape_constraint.

related: an inherited attribute that shall be of type Occurrence_shape_feature.

Formal propositions:

WR1: Following two paths starting from the related and relating attributes shall end in the same instance of Assembly_definition. The first Assembly_definition is reached by following the associated_definition of the relating Assembly_shape_joint. The second Assembly_definition is reached by following the associated_definition of the related Occurrence_shape_feature through an optional chain of Specified_occurrence to a top Product_occurrence that is used in the Assembly_definition by a Next_assembly_usage.

## 4.3.3 Assembly_shape_joint

An Assembly_shape_joint is a type of Part_shape_element that defines a joint between two or more Occurrence_shape_features within anAssembly_definition. An Assembly_shape_joint may be composed of other Assembly_shape_joints.

EXPRESS specification:

*)
ENTITY Assembly_shape_joint
  SUBTYPE OF (Part_shape_element);
  SELF\shape_element.associated_definition : assembly_definition_or_assembly_shape_joint;
  SELF\part_shape_element.definition : OPTIONAL assembly_shape_constraint_or_joint_definition_select;
  joint_type : OPTIONAL general_assembly_joint_type_enumeration;
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Assembly_definition or of type Assembly_shape_joint.

definition: a Contact_feature_definition_fit_relationship or a Shape_feature_definition that is the definition for the Assembly_shape_joint. The value of this attribute need not be specified.

joint_type: specifies the way how the joint is realised. The value of this attribute need not be specified.

## 4.3.4 Assembly_shape_joint_item_relationship

An Assembly_shape_joint_item_relationship is a type of Shape_element_relationship that relates an Occurrence_shape_feature into an Assembly_shape_constraint.

EXPRESS specification:

*)
ENTITY Assembly_shape_joint_item_relationship
  SUBTYPE OF (Shape_element_relationship);
  SELF\shape_element_relationship.relating : Assembly_shape_joint;
  SELF\shape_element_relationship.related : Occurrence_shape_feature;
WHERE
  WR1: main_associated_definition(relating\Assembly_shape_joint.associated_definition) IN main_upper_usage(main_associated_definition(related))\Product_occurrence.occurrence_contexts;
END_ENTITY;
(*

Attribute definitions:

relating: an inherited attribute that shall be of type Assembly_shape_joint.

related: an inherited attribute that shall be of type Occurrence_shape_feature.

Formal propositions:

WR1: Following two paths starting from the related and relating attributes shall end in the same instance of Assembly_definition. The first Assembly_definition is reached by following the associated_definition of the relating Assembly_shape_joint. The second Assembly_definition is reached by following the associated_definition of the related Occurrence_shape_feature through an optional chain of Specified_occurrence to a top Product_occurrencethat is used in the Assembly_definition by a Next_assembly_usage.

## 4.3.5 Contact_feature_definition

A Contact_feature_definition is a type of Shape_feature_definition that can be connected with another Contact_feature_definition in the case that a Contact_feature_definition_fit_relationship is defined between them.

EXPRESS specification:

*)
ENTITY Contact_feature_definition
  SUBTYPE OF (Shape_feature_definition);
END_ENTITY;
(*

## 4.3.6 Contact_feature_definition_fit_relationship

A Contact_feature_definition_fit_relationship is a type of Shape_feature_definition_fit_relationship that is a relationship between two instances of the entity data type Contact_feature_definition that fit together.

EXPRESS specification:

*)
ENTITY Contact_feature_definition_fit_relationship
  SUBTYPE OF (Shape_feature_definition_fit_relationship);
  SELF\shape_feature_definition_relationship.relating : Contact_feature_definition;
  SELF\shape_feature_definition_relationship.related : Contact_feature_definition;
  joint_type : OPTIONAL general_assembly_joint_type_enumeration;
END_ENTITY;
(*

Attribute definitions:

relating: one of the instances of Contact_feature_definition that is a part of the relationship.

related: the other instance of Contact_feature_definition that is a part of the relationship. If one element of the relationship is dependent upon the other, this attribute shall be the dependent one.

joint_type: the intended joint type for this Contact_feature_definition_fit_relationship. The value of this attribute need not be specified.

## 4.3.7 General_part_terminal

A General_part_terminal is a type of Part_contact_feature that can be used within an assembly as the definition of an Occurrence_terminal that can be joined with another Occurrence_terminal to transport information, energy, or matter.

EXPRESS specification:

*)
ENTITY General_part_terminal
  SUBTYPE OF (Part_contact_feature);
  SELF\shape_element.associated_definition : general_part_terminal_associated_definition_select;
  associated_transport_feature : OPTIONAL Part_transport_feature;
  intended_joint_type : OPTIONAL SET[1:?] OF general_terminal_joint_type_enumeration;
  join_or_interface_terminal : OPTIONAL join_or_interface_terminal_enumeration;
  domain_type : OPTIONAL terminal_and_transport_domain_type;
WHERE
  WR1: main_associated_definition(SELF) :=: main_associated_definition(associated_transport_feature);
  WR2: domain_type :=: associated_transport_feature.domain_type;
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Part_view_definition or of type General_part_terminal.

associated_transport_feature: the Part_transport_feature to which the terminal is connected to. The value of this attribute need not be specified.

intended_joint_type: the intended joint type of the terminal. The value of this attribute need not be specified.

join_or_interface_terminal: an indication on whether the terminal is intended to be connected at the immediate-assembly level or a next-higher-assembly level. The value of this attribute need not be specified.

domain_type: the domain type of the General_part_terminal. The value of this attribute need not be specified.

Formal propositions:

WR1: The optional chain of associated_definitions of the General_part_terminal shall end in the same Part_view_definition as the optional chain of associated_definitions of the associated_transport_features.

WR2: The optional domain_type shall have the same value as the one from the associated_transport_feature if defined.

## 4.3.8 Occurrence_contact_feature

An Occurrence_contact_feature is a type of Occurrence_shape_feature that is intended to be joined with another Occurrence_contact_feature.

EXPRESS specification:

*)
ENTITY Occurrence_contact_feature
  SUPERTYPE OF (Occurrence_terminal)
  SUBTYPE OF (Occurrence_shape_feature);
  SELF\shape_element.associated_definition : occurrence_contact_feature_associated_definition_select;
  SELF\occurrence_shape_feature.definition : OPTIONAL occurrence_contact_feature_definition_select;
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Product_occurrence or of type Occurrence_contact_feature.

definition: an inherited attribute that shall have a value of type Occurrence_contact_feature, Part_contact_feature or Contact_feature_definition. The value of this attribute need not be specified.

## 4.3.9 Occurrence_shape_element

An Occurrence_shape_element is a type of Shape_element that represents a part of the shape of a Product_occurrence or another Occurrence_shape_element.

EXPRESS specification:

*)
ENTITY Occurrence_shape_element
  SUBTYPE OF (Shape_element);
  SELF\shape_element.associated_definition : occurrence_shape_element_associated_definition_select;
  definition : OPTIONAL occurrence_shape_element_definition_select;
UNIQUE
  UR1: definition, associated_definition;
  UR2: SELF\shape_element.element_name, associated_definition;
WHERE
  WR1: acyclic_occurrence_shape_element(SELF, definition);
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Product_occurrence or of type Occurrence_shape_element.

definition: an inherited attribute that shall have a value of type Part_shape_element, Occurrence_shape_element or Shape_feature_definition. The value of this attribute need not be specified.

Formal propositions:

UR1: There shall be at most one Occurrence_shape_element of a definition for an associated_definition.

UR2: The element_name of the Occurrence_shape_element shall be unique for the associated_definition.

WR1: A list of Occurrence_shape_element instances linked by their definition attribute values shall not be cyclic.

## 4.3.10 Occurrence_shape_feature

An Occurrence_shape_feature is a type of Occurrence_shape_element that is a definitional part of the shape of a Product_occurrence or another Occurrence_shape_feature.

EXPRESS specification:

*)
ENTITY Occurrence_shape_feature
  SUPERTYPE OF (ONEOF (Occurrence_contact_feature,
                       Occurrence_transport_feature))
  SUBTYPE OF (Occurrence_shape_element);
  SELF\shape_element.associated_definition : occurrence_shape_feature_associated_definition_select;
  SELF\occurrence_shape_element.definition : OPTIONAL occurrence_shape_feature_definition_select;
DERIVE
  SELF\shape_element.product_definitional : BOOLEAN := TRUE;
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Product_occurrence or of type Occurrence_shape_feature.

definition: an inherited attribute that shall have a value of type General_part_feature, Occurrence_shape_feature or Shape_feature_definition. The value of this attribute need not be specified.

product_definitional: an inherited attribute that shall have the value TRUE.

## 4.3.11 Occurrence_terminal

An Occurrence_terminal is a type of Occurrence_contact_feature that can be joined with another Occurrence_terminal for transporting information, energy, or matter.

EXPRESS specification:

*)
ENTITY Occurrence_terminal
  SUBTYPE OF (Occurrence_contact_feature);
  SELF\shape_element.associated_definition : occurrence_terminal_associated_definition_select;
  SELF\occurrence_shape_element.definition : occurrence_terminal_definition_select;
  associated_transport_feature : OPTIONAL Occurrence_transport_feature;
WHERE
  WR1: main_associated_definition(associated_transport_feature.associated_definition) :=: main_associated_definition(associated_definition);
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Product_occurrence or of type Occurrence_terminal.

definition: an inherited attribute that shall have a value of type General_part_terminal or of type Occurrence_terminal.

associated_transport_feature: the Occurrence_transport_feature to which the terminal is connected. The value of this attribute need not be specified.

Formal propositions:

WR1: In the case that the associated_transport_feature is defined then the reference Occurrence_transport_feature shall refer to the same Product_occurrence as the Occurrence_terminal.

## 4.3.12 Occurrence_transport_feature

An Occurrence_transport_feature is a type of Occurrence_shape_feature that enables the transport of information, energy, or matter between several Occurrence_terminals of the same Product_occurrence.

EXPRESS specification:

*)
ENTITY Occurrence_transport_feature
  SUBTYPE OF (Occurrence_shape_feature);
  SELF\shape_element.associated_definition : occurrence_transport_feature_associated_definition_select;
  SELF\occurrence_shape_element.definition : OPTIONAL occurrence_transport_feature_definition_select;
  domain_type : OPTIONAL terminal_and_transport_domain_type;
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Product_occurrence or of type Occurrence_transport_feature.

definition: an inherited attribute that shall have a value of type Part_transport_feature or another Occurrence_transport_feature. The value of this attribute need not be specified.

domain_type: the domain type of the Occurrence_transport_feature. The value of this attribute need not be specified.

## 4.3.13 Part_connectivity_definition

A Part_connectivity_definition is a type of Part_shape_element that defines a connection between two or more General_part_terminals or Occurrence_terminals within an Assembly_definition but without stating the details how these terminals are connected with each other.

EXPRESS specification:

*)
ENTITY Part_connectivity_definition
  SUBTYPE OF (Part_shape_element);
  connected_terminals : SET[2:?] OF connected_terminal_select;
WHERE
  WR1: terminal_usage_check(main_associated_definition(associated_definition), connected_terminals);
END_ENTITY;
(*

Attribute definitions:

connected_terminals: the set of General_part_terminals or Occurrence_terminals that are connected with each other.

Formal propositions:

WR1: Each terminal in the set of connected_terminals shall be accessible within the Part_view_definition or Assembly_definition that can be reached by the associated_definition. In the case the terminal is of type Occurrence_terminal then a potential chain of associated_definitions must be followed till a Product_occurrence is reached. If this Product_occurrence is of type Specified_occurrence, a potential chain of upper_usages is to be followed to a top Product_occurrence that shall be related to the same Assembly_definition by a Next_assembly_usage. In the case, the terminal is of type General_part_terminal then following a potential chain of associated_definitions shall lead to the same Assembly_definition.

## 4.3.14 Part_contact_feature

A Part_contact_feature is a type of General_part_feature that is intended to be joined with another General_part_feature when used in an assembly.

EXPRESS specification:

*)
ENTITY Part_contact_feature
  SUBTYPE OF (General_part_feature);
  SELF\shape_element.associated_definition : part_contact_feature_associated_definition_select;
  SELF\part_shape_element.definition : OPTIONAL part_contact_feature_definition_select;
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Part_view_definition or of type Part_contact_feature.

definition: the Contact_feature_definition or Shape_feature_definition_element that defines the Part_contact_feature. The value of this attribute need not be specified.

## 4.3.15 Part_contact_feature_fit_relationship

A Part_contact_feature_fit_relationship is a type of Part_feature_fit_relationship that is a relationship between two instances of the entity data type Part_contact_feature stating that usages of these in an assembly can be joined with each other.

NOTE    The associated_definition of the related and relating Part_contact_features may be the same or different Part_view_definitions.

EXPRESS specification:

*)
ENTITY Part_contact_feature_fit_relationship
  SUBTYPE OF (Part_feature_fit_relationship);
  SELF\shape_element_relationship.related : Part_contact_feature;
  SELF\shape_element_relationship.relating : Part_contact_feature;
  SELF\part_feature_fit_relationship.definition : OPTIONAL part_contact_feature_fit_relationship_definition_select;
  parent_relationship : OPTIONAL Part_contact_feature_fit_relationship;
WHERE
  WR1: ((related.definition :=: definition\Shape_feature_definition_relationship.related) OR (related.definition :=: definition\Shape_feature_definition_element_relationship.related)) AND ((relating.definition :=: definition\Shape_feature_definition_relationship.relating) OR (relating.definition :=: definition\Shape_feature_definition_element_relationship.relating));
  WR2: (related.associated_definition :=: parent_relationship.related) AND (relating.associated_definition :=: parent_relationship.relating);
END_ENTITY;
(*

Attribute definitions:

related: the other instance of Part_contact_feature that is a part of the relationship. If one element of the relationship is dependent upon the other, this attribute shall be the dependent one.

relating: one of the instances of Part_contact_feature that is a part of the relationship.

definition: the Contact_feature_definition_fit_relationship or Shape_feature_definition_element_relationship that defines the Part_contact_feature_fit_relationship. The value of this attribute need not be specified.

parent_relationship: this Part_contact_feature_fit_relationship is a member of the higher-level Part_contact_feature_fit_relationship. The value of this attribute need not be specified.

Formal propositions:

WR1: if a local definition is defined, then the definition of the related Part_contact_feature shall be the same instance as the related instance of the local definition, and the definition of the relating Part_contact_feature shall be the same instance as the relating instance of the local definition.

WR2: if the parent_relationship is defined, then the associated_definition of the related Part_contact_feature shall be the same instance as the related instance of the parent_relationship, and the associated_definition of the relating Part_contact_feature shall be the same instance as the relating instance of the parent_relationship.

## 4.3.16 Part_feature_fit_relationship

A Part_feature_fit_relationship is a type of Part_shape_element_relationship that states that the two related General_part_features fit with each other.

EXPRESS specification:

*)
ENTITY Part_feature_fit_relationship
  SUBTYPE OF (Part_shape_element_relationship);
  SELF\shape_element_relationship.related : General_part_feature;
  SELF\shape_element_relationship.relating : General_part_feature;
  definition : OPTIONAL part_feature_fit_relationship_definition_select;
END_ENTITY;
(*

Attribute definitions:

related: an inherited attribute whose value shall be of type General_part_feature.

relating: an inherited attribute whose value shall be of type General_part_feature.

definition: the Shape_feature_definition_fit_relationship or Shape_feature_definition_element_relationship that defines the Part_feature_fit_relationship.

## 4.3.17 Part_transport_feature

A Part_transport_feature is a type of General_part_feature that can transport information, energy, or matter within the Part_view_definition.

EXPRESS specification:

*)
ENTITY Part_transport_feature
  SUBTYPE OF (General_part_feature);
  SELF\shape_element.associated_definition : part_transport_feature_associated_definition_select;
  domain_type : OPTIONAL terminal_and_transport_domain_type;
END_ENTITY;
(*

Attribute definitions:

associated_definition: an inherited attribute that shall have a value of type Part_view_definition. or of type Part_transport_feature.

domain_type: the domain type of the Part_transport_feature. The value of this attribute need not be specified.

## 4.3.18 Shape_feature_definition_fit_relationship

A Shape_feature_definition_fit_relationship is a type of Shape_feature_definition_relationship that state that the two related Shape_feature_definition fit with each other.

EXPRESS specification:

*)
ENTITY Shape_feature_definition_fit_relationship
  SUBTYPE OF (Shape_feature_definition_relationship);
END_ENTITY;
(*

## 4.4 ARM subtype constraint definitions

This subclause specifies the ARM subtype constraints for this module. Each subtype constraint places constraints on the possible super-type / subtype instantiations. The ARM subtype constraints and definitions are specified below.

## 4.4.1 gdc_general_part_feature_subtypes

The gdc_general_part_feature_subtypes constraint specifies that instances of subtypes of general_part_feature shall not be simultaneously of type Part_transport_feature, and Part_contact_feature.

EXPRESS specification:

*)
SUBTYPE_CONSTRAINT gdc_general_part_feature_subtypes FOR general_part_feature;
  ONEOF (Part_transport_feature,
         Part_contact_feature);
END_SUBTYPE_CONSTRAINT;
(*

## 4.4.2 gdc_part_shape_element_subtypes

The gdc_part_shape_element_subtypes constraint specifies that instances of subtypes of part_shape_element shall not be simultaneously of type Assembly_shape_constraint, Assembly_shape_joint, and Part_connectivity_definition.

EXPRESS specification:

*)
SUBTYPE_CONSTRAINT gdc_part_shape_element_subtypes FOR part_shape_element;
  ONEOF (Assembly_shape_constraint,
         Assembly_shape_joint,
         Part_connectivity_definition);
END_SUBTYPE_CONSTRAINT;
(*

## 4.4.3 gdc_shape_element_subtypes

The gdc_shape_element_subtypes constraint specifies that instances of subtypes of shape_element shall not be simultaneously of type Occurrence_shape_element, Part_shape_element, and Shape_feature_definition_element.

EXPRESS specification:

*)
SUBTYPE_CONSTRAINT gdc_shape_element_subtypes FOR shape_element;
  ONEOF (Occurrence_shape_element,
         Part_shape_element,
         Shape_feature_definition_element);
END_SUBTYPE_CONSTRAINT;
(*

## 4.5 ARM function definitions

This subclause specifies the ARM functions for this module. The ARM functions and definitions are specified below.

## 4.5.1 acyclic_occurrence_shape_element

The acyclic_occurrence_shape_element function determines whether the graph of instances of the entity data type Occurrence_shape_element established by the attribute definition contains a cycle. The function returns TRUE if no cycle has been detected. Otherwise, it returns FALSE.

EXPRESS specification:

*)
FUNCTION acyclic_occurrence_shape_element (sao : Occurrence_shape_element; definition : occurrence_shape_element_definition_select) : BOOLEAN;

IF NOT (('GENERAL_DESIGN_CONNECTIVITY_ARM.OCCURRENCE_SHAPE_ELEMENT') IN TYPEOF(definition))

THEN

RETURN(TRUE);

END_IF;

IF (definition :=: sao)

THEN

RETURN(FALSE);

ELSE

RETURN(acyclic_occurrence_shape_element(sao, definition\Occurrence_shape_element.definition));

END_IF;

END_FUNCTION; 
(*

Argument definitions:

sao: (input) the start node of type Occurrence_shape_element to be checked.

definition: (input) the next node to which the start node is checked.

## 4.5.2 acyclic_shape_element

The acyclic_shape_element function determines whether the graph of instances of the entity data type Shape_element established by the attribute associated_definition contains a cycle. The function returns TRUE if no cycle has been detected. Otherwise, it returns FALSE.

EXPRESS specification:

*)
FUNCTION acyclic_shape_element (se : Shape_element; associated_definition : occurrence_shape_element_definition_select) : BOOLEAN;

IF NOT (('SHAPE_PROPERTY_ASSIGNMENT_ARM.SHAPE_ELEMENT') IN TYPEOF(associated_definition))

THEN

RETURN(TRUE);

END_IF;

IF (associated_definition :=: se)

THEN

RETURN(FALSE);

ELSE

RETURN(acyclic_shape_element(se, associated_definition\Shape_element.associated_definition));

END_IF;

END_FUNCTION; 
(*

Argument definitions:

se: (input) the start node of type Shape_element to be checked.

associated_definition: (input) the next node to which the start node is checked.

## 4.5.3 main_associated_definition

The main_associated_definition function determines the final associated_definition of a potential list of instances of the entity data type Shape_element. The function returns the final object that is not of type Shape_element.

EXPRESS specification:

*)
FUNCTION main_associated_definition (se : shapeable_item) : shapeable_item;

IF NOT (('SHAPE_PROPERTY_ASSIGNMENT.SHAPE_ELEMENT') IN TYPEOF(se))

THEN

RETURN(se);

ELSE

IF NOT (('SHAPE_PROPERTY_ASSIGNMENT.SHAPE_ELEMENT') IN TYPEOF(se.associated_definition))

THEN

RETURN(se.associated_definition);

ELSE

RETURN(main_associated_definition(se.associated_definition));

END_IF;

END_IF;

END_FUNCTION; 
(*

Argument definitions:

se: (input) the start node of type shapeable_item.

## 4.5.4 main_upper_usage

The main_upper_usage function determines the final instance of a potential list of instances of entity data type Specified_occurrence that are linked by the attribute upper_usage. The function returns the final Product_occurrence that is not of type Specified_occurrence.

EXPRESS specification:

*)
FUNCTION main_upper_usage (po : Product_occurrence) : Product_occurrence;

IF NOT (('PRODUCT_OCCURRENCE_ARM.SPECIFIED_OCCURRENCE') IN TYPEOF(po))

THEN

RETURN(po);

ELSE

RETURN(main_upper_usage(po\Specified_occurrence.upper_usage));

END_IF;

END_FUNCTION; 
(*

Argument definitions:

po: (input) the start node of type Product_occurrence.

## 4.5.5 terminal_usage_check

The terminal_usage_check function determines if a set of terminals is accessible for a Part_view_definition. In the case where a terminal in the set of terminals is of type Occurrence_terminal then a potential chain of associated_definitions must be followed till a Product_occurrence is reached. If this Product_occurrence is of type Specified_occurrence, a potential chain of upper_usages is to be followed to a top Product_occurrence that shall be related to the same Assembly_definition by a Next_assembly_usage. In the case, the terminal is of type General_part_terminal then following a potential chain of associated_definitions shall lead to the same Assembly_definition. The function returns TRUE if all terminals are accessible to the Part_view_definition. Otherwise, it returns FALSE.

EXPRESS specification:

*)
FUNCTION terminal_usage_check (p : Part_view_definition; terminals : SET OF connected_terminal_select) : BOOLEAN;

REPEAT i := 1 TO HIINDEX(terminals) BY 1;

IF 'GENERAL_DESIGN_CONNECTIVITY_ARM.GENERAL_PART_TERMINAL' IN TYPEOF(terminals[i])

THEN

IF p :<>: terminals[i]\General_part_terminal.associated_definition

THEN

RETURN(FALSE);

END_IF;

ELSE

IF 'GENERAL_DESIGN_CONNECTIVITY_ARM.OCCURRENCE_TERMINAL' IN TYPEOF(terminals[i])

THEN

IF NOT (p = main_upper_usage(main_associated_definition(terminals[i])))

THEN

RETURN(FALSE);

END_IF;

ELSE

RETURN(FALSE);

END_IF;

END_IF;

END_REPEAT;

RETURN(TRUE);

END_FUNCTION; 
(*

Argument definitions:

p: (input) the Part_view_definition against which the set of terminals is checked.

terminals: (input) the set of terminals that are either of type Occurrence_terminal or of type General_part_terminal.

## 4.6 ARM rule definition

This subclause specifies the ARM rule for this module. The ARM rule and definition is specified below.

## 4.6.1 shape_elements_are_acyclic

The shape_elements_are_acyclic rule constrains any instance of type Shape_element to not participate in a cycle established by the attribute associated_definition.

EXPRESS specification:

*)
RULE shape_elements_are_acyclic FOR 
(shape_element);
WHERE
  WR1: SIZEOF(QUERY(se <* shape_element | NOT acyclic_shape_element(se, se.associated_definition))) = 0;
END_RULE; 
(*

Argument definitions:

shape_element : the set of all instances of shape_element.

Formal propositions:

WR1: A list of Shape_element instances linked by their associated_definition attribute values shall not be cyclic.

*)
END_SCHEMA;  -- General_design_connectivity_arm
(*

© ISO 2019 — All rights reserved