## 5.2 MIM EXPRESS short listing

This clause specifies the EXPRESS schema derived from the mapping table. It uses elements from the common resources or from other application modules and defines the EXPRESS constructs that are specific to this part of ISO 10303.

This clause constitutes the Module Interpreted Module (MIM) of the application module.

This clause also specifies the modifications that apply to the constructs imported from the common resources.

The following restrictions apply to the use, in this schema, of constructs defined in common resources or in application modules:

- use of a supertype entity does not make applicable any of its specializations, unless the specialization is also imported in the MIM schema;

- use of a SELECT type does not make applicable any of its listed types unless the listed type is also imported in the MIM schema.

Short names of entities defined in this schema are described in Annex A. Unambiguous identification of this schema is defined in Annex B.

EXPRESS specification: 

*)
SCHEMA General_design_connectivity_mim;

USE FROM Assembly_constraints_mim;    --  ISO/TS 10303-1794

USE FROM Design_product_data_management_mim;    --  ISO/TS 10303-1628

USE FROM Item_definition_structure_mim;    --  ISO/TS 10303-1345

USE FROM Part_shape_mim;    --  ISO/TS 10303-1807

USE FROM Shape_feature_mim;    --  ISO/TS 10303-1764
(*

NOTE 1   The schemas referenced above are specified in the following part of ISO 10303:

| Assembly_constraints_mim | ISO/TS 10303-1794 |
| --- | --- |
| Design_product_data_management_mim | ISO/TS 10303-1628 |
| Item_definition_structure_mim | ISO/TS 10303-1345 |
| Part_shape_mim | ISO/TS 10303-1807 |
| Shape_feature_mim | ISO/TS 10303-1764 |

NOTE 2   See Annex D, Figures D.1and D.2 for a graphical representation of this schema.

## 5.2.1 MIM entity definitions

This subclause specifies the MIM entities for this module. The MIM entities and definitions are specified below.

## 5.2.1.1 assembly_shape_constraint

An assembly_shape_constraint is a type of shape_aspect that implements parts of the ARM concept of Assembly_shape_constraint.

EXPRESS specification:

*)
ENTITY assembly_shape_constraint
  SUBTYPE OF (shape_aspect);
END_ENTITY;
(*

## 5.2.1.2 assembly_shape_constraint_item_relationship

An assembly_shape_constraint_item_relationship is a type of shape_aspect_relationship that implements parts of the ARM concept of Assembly_shape_constraint_item_relationship.

EXPRESS specification:

*)
ENTITY assembly_shape_constraint_item_relationship
  SUBTYPE OF (shape_aspect_relationship);
  SELF\shape_aspect_relationship.relating_shape_aspect : assembly_shape_constraint;
END_ENTITY;
(*

Attribute definitions:

relating_shape_aspect: an inherited attribute that shall have a value of type assembly_shape_constraint.

## 5.2.1.3 assembly_shape_joint

An assembly_shape_joint is a type of shape_aspect that implements parts of the ARM concept of Assembly_shape_joint.

EXPRESS specification:

*)
ENTITY assembly_shape_joint
  SUBTYPE OF (shape_aspect);
END_ENTITY;
(*

## 5.2.1.4 assembly_shape_joint_item_relationship

An assembly_shape_joint_item_relationship is a type of shape_aspect_relationship that implements parts of the ARM concept of Assembly_shape_joint_item_relationship.

EXPRESS specification:

*)
ENTITY assembly_shape_joint_item_relationship
  SUBTYPE OF (shape_aspect_relationship);
  SELF\shape_aspect_relationship.relating_shape_aspect : assembly_shape_joint;
END_ENTITY;
(*

Attribute definitions:

relating_shape_aspect: an inherited attribute that shall have a value of type assembly_shape_joint.

## 5.2.1.5 connectivity_definition

A connectivity_definition is a type of shape_aspect that implements the ARM concept of Part_connectivity_definition.

EXPRESS specification:

*)
ENTITY connectivity_definition
  SUBTYPE OF (shape_aspect);
END_ENTITY;
(*

## 5.2.1.6 connectivity_definition_item_relationship

A connectivity_definition_item_relationship is a type of shape_aspect_relationship that implements parts of the ARM concept of Part_connectivity_definition for the attribute connected_terminals.

EXPRESS specification:

*)
ENTITY connectivity_definition_item_relationship
  SUBTYPE OF (shape_aspect_relationship);
  SELF\shape_aspect_relationship.relating_shape_aspect : connectivity_definition;
END_ENTITY;
(*

Attribute definitions:

relating_shape_aspect: an inherited attribute that shall have a value of type connectivity_definition.

## 5.2.1.7 contact_feature

A contact_feature is a type of shape_aspect that implements parts of the ARM concept of Occurrence_contact_feature and Part_contact_feature.

EXPRESS specification:

*)
ENTITY contact_feature
  SUBTYPE OF (shape_aspect);
END_ENTITY;
(*

## 5.2.1.8 contact_feature_definition

A contact_feature_definition is a type of shape_feature_definition that implements parts of the ARM concept of Contact_feature_definition.

EXPRESS specification:

*)
ENTITY contact_feature_definition
  SUBTYPE OF (shape_feature_definition);
END_ENTITY;
(*

## 5.2.1.9 contact_feature_definition_fit_relationship

A contact_feature_definition_fit_relationship is a type of group_assignment that implements parts of the ARM concept of Contact_feature_definition_fit_relationship.

EXPRESS specification:

*)
ENTITY contact_feature_definition_fit_relationship
  SUBTYPE OF (shape_feature_definition_fit_relationship);
  SELF\characterized_object_relationship.relating_object : contact_feature_definition;
  SELF\characterized_object_relationship.related_object : contact_feature_definition;
END_ENTITY;
(*

Attribute definitions:

relating_object: an inherited attribute that shall have a value of type contact_feature_definition.

related_object: an inherited attribute that shall have a value of type contact_feature_definition.

## 5.2.1.10 contact_feature_fit_relationship

A contact_feature_fit_relationship is a type of shape_feature_fit_relationship that implements parts of the ARM concept of Part_contact_feature_fit_relationship.

EXPRESS specification:

*)
ENTITY contact_feature_fit_relationship
  SUBTYPE OF (shape_feature_fit_relationship);
  SELF\shape_aspect_relationship.relating_shape_aspect : contact_feature;
  SELF\shape_aspect_relationship.related_shape_aspect : contact_feature;
END_ENTITY;
(*

Attribute definitions:

relating_shape_aspect: an inherited attribute that shall have a value of type contact_feature.

related_shape_aspect: an inherited attribute that shall have a value of type contact_feature.

## 5.2.1.11 shape_feature_definition_fit_relationship

A shape_feature_definition_fit_relationship is a type of contact_feature that implements parts of the ARM concept of Shape_feature_definition_fit_relationship.

EXPRESS specification:

*)
ENTITY shape_feature_definition_fit_relationship
  SUBTYPE OF (shape_feature_definition_relationship);
END_ENTITY;
(*

## 5.2.1.12 shape_feature_fit_relationship

A shape_feature_fit_relationship is a type of shape_aspect_relationship and of characterized_object that implements parts of the ARM concept of Part_feature_fit_relationship

EXPRESS specification:

*)
ENTITY shape_feature_fit_relationship
  SUBTYPE OF (shape_aspect_relationship, characterized_object);
DERIVE
  SELF\characterized_object.name : label := SELF\shape_aspect_relationship.name;
  SELF\characterized_object.description : text := SELF\shape_aspect_relationship.description;
END_ENTITY;
(*

Attribute definitions:

name: an inherited attribute of characterized_object that shall have the same value as the corresponding attribute inherited from shape_aspect_relationship.

description: an inherited attribute of characterized_object that shall have the same value as the corresponding attribute inherited from shape_aspect_relationship.

## 5.2.1.13 terminal_feature

A terminal_feature is a type of contact_feature that implements parts of the ARM concept of General_part_terminal and Occurrence_terminal.

EXPRESS specification:

*)
ENTITY terminal_feature
  SUBTYPE OF (contact_feature);
END_ENTITY;
(*

## 5.2.1.14 transport_feature

A transport_feature is a type of shape_aspect that implements the ARM concept of Occurrence_transport_feature and Part_transport_feature.

EXPRESS specification:

*)
ENTITY transport_feature
  SUBTYPE OF (shape_aspect);
END_ENTITY;
(*

## 5.2.2 MIM subtype constraint definitions

This subclause specifies the MIM subtype constraints for this module. Each subtype constraint places constraints on the possible super-type / subtype instantiations. The MIM subtype constraints and definitions are specified below.

## 5.2.2.1 gdc_shape_aspect_subtypes

The gdc_shape_aspect_subtypes constraint specifies a constraint that applies to instances of shape_aspect and enforces the rule that its subtypesassembly_shape_constraint, assembly_shape_joint, connectivity_definition, contact_feature and transport_feature are exclusive.

EXPRESS specification:

*)
SUBTYPE_CONSTRAINT gdc_shape_aspect_subtypes FOR shape_aspect;
  ONEOF (assembly_shape_constraint,
         assembly_shape_joint,
         connectivity_definition,
         contact_feature,
         transport_feature);
END_SUBTYPE_CONSTRAINT;
(*

## 5.2.2.2 gdc_shape_aspect_relationship_subtypes

The gdc_shape_aspect_relationship_subtypes constraint specifies a constraint that applies to instances of shape_aspect_relationship and enforces the rule that its subtypes assembly_shape_joint_item_relationship, assembly_shape_constraint_item_relationship, connectivity_definition_item_relationship and shape_feature_fit_relationship are exclusive.

EXPRESS specification:

*)
SUBTYPE_CONSTRAINT gdc_shape_aspect_relationship_subtypes FOR shape_aspect_relationship;
  ONEOF (assembly_shape_joint_item_relationship,
         assembly_shape_constraint_item_relationship,
         connectivity_definition_item_relationship,
         shape_feature_fit_relationship);
END_SUBTYPE_CONSTRAINT;
(*

*)
END_SCHEMA;  -- General_design_connectivity_mim
(*

© ISO 2019 — All rights reserved